软著源代码（第二部分）

PCB缺陷检测与智能分拣系统 V1.0  |  源代码  |  共 2407 行

1  
    2  # ============================================================
    3  # 文件: ui/gui.py
    4  # 说明: PyQt5图形用户界面 — 实时预览、检测结果、专家评分、统计仪表盘
    5  # 行数: 1689
    6  # ============================================================
    7  # -*- coding: utf-8 -*-
    8  """
    9  PyQt5 图形用户界面
   10  功能：
   11  1. 实时视频预览
   12  2. 检测结果显示（带边界框）
   13  3. 专家评分模式（评委逐张判分）
   14  4. 历史记录统计
   15  5. 系统状态监控
   16  6. 急停按钮
   17  7. 统计仪表盘（合格率、缺陷类别分布、平均耗时）
   18  8. CSV报表导出
   19  9. 摄像头断开重连机制
   20  10. 一键换线（产线配置热切换）
   21  11. 误报屏蔽（区域忽略规则）
   22  12. 缺陷追溯（SN码/工号/设备号）
   23  13. 产线/调试模式切换
   24  14. 批量复核
   25  15. 设备健康度监控
   26  16. 声光报警
   27  17. 生产报表
   28  18. 权限管理
   29  """
   30  
   31  import os
   32  import sys
   33  import time
   34  import csv
   35  import json
   36  import re
   37  from pathlib import Path
   38  from datetime import datetime
   39  from collections import deque, Counter
   40  
   41  import cv2
   42  import numpy as np
   43  from PyQt5.QtWidgets import (
   44      QApplication, QMainWindow, QWidget, QLabel, QPushButton,
   45      QVBoxLayout, QHBoxLayout, QGridLayout, QFrame,
   46      QTableWidget, QTableWidgetItem, QHeaderView,
   47      QMessageBox, QFileDialog, QGroupBox, QSplitter,
   48      QStatusBar, QProgressBar, QSpinBox, QDoubleSpinBox,
   49      QCheckBox, QComboBox, QTextEdit, QTabWidget,
   50      QLineEdit, QDialog, QFormLayout, QDialogButtonBox,
   51      QListWidget, QListWidgetItem, QMenu, QAction, QInputDialog,
   52  )
   53  from PyQt5.QtCore import Qt, QTimer, QThread, pyqtSignal, QMutex, QWaitCondition, QRect
   54  from PyQt5.QtGui import QImage, QPixmap, QFont, QColor, QPalette, QPainter, QPen, QBrush
   55  
   56  sys.path.append(os.path.dirname(os.path.dirname(os.path.abspath(__file__))))
   57  from utils.config_loader import get_config
   58  from utils.logger import setup_logger
   59  from utils.production_manager import ProductionManager, LineProfile
   60  from utils.false_alarm_suppressor import FalseAlarmSuppressor, SuppressionRule
   61  from utils.traceability import TraceabilityDB
   62  from utils.alarm import AlarmManager, AlarmConfig, AlarmLevel
   63  from utils.equipment_monitor import EquipmentMonitor, HealthStatus
   64  from utils.auth import AuthManager, Role
   65  from utils.production_report import ProductionReportGenerator
   66  
   67  logger = setup_logger("gui")
   68  
   69  
   70  class CameraThread(QThread):
   71      """摄像头采集线程 - 带断开重连机制"""
   72  
   73      frame_ready = pyqtSignal(np.ndarray)
   74      error_occurred = pyqtSignal(str)
   75      connection_lost = pyqtSignal()
   76      connection_restored = pyqtSignal()
   77  
   78      def __init__(self, camera_id=0, width=1920, height=1080, fps=30):
   79          super().__init__()
   80          self.camera_id = camera_id
   81          self.width = width
   82          self.height = height
   83          self.fps = fps
   84          self.running = False
   85          self.cap = None
   86          self._mutex = QMutex()
   87          self._reconnect_interval = 3.0
   88          self._max_reconnect_attempts = 0
   89          self._connected = False
   90  
   91      def run(self):
   92          self.running = True
   93  
   94          while self.running:
   95              if not self._open_camera():
   96                  self.error_occurred.emit(f"无法打开摄像头: {self.camera_id}")
   97                  self._connected = False
   98                  self.connection_lost.emit()
   99  
  100                  attempt = 0
  101                  while self.running:
  102                      attempt += 1
  103                      logger.info(f"摄像头重连尝试 #{attempt}...")
  104                      time.sleep(self._reconnect_interval)
  105  
  106                      if not self.running:
  107                          break
  108  
  109                      if self._open_camera():
  110                          self._connected = True
  111                          self.connection_restored.emit()
  112                          logger.info(f"摄像头重连成功 (第{attempt}次尝试)")
  113                          break
  114  
  115                      if self._max_reconnect_attempts > 0 and attempt >= self._max_reconnect_attempts:
  116                          self.error_occurred.emit(f"摄像头重连失败，已达最大尝试次数({self._max_reconnect_attempts})")
  117                          return
  118                  continue
  119  
  120              self._connected = True
  121              self.connection_restored.emit()
  122  
  123              consecutive_failures = 0
  124              while self.running:
  125                  ret, frame = self.cap.read()
  126                  if ret:
  127                      consecutive_failures = 0
  128                      self.frame_ready.emit(frame)
  129                  else:
  130                      consecutive_failures += 1
  131                      if consecutive_failures >= 10:
  132                          logger.warning("摄像头连续读取失败，判定为断开连接")
  133                          self._connected = False
  134                          self.connection_lost.emit()
  135                          self._release_camera()
  136                          break
  137  
  138                  time.sleep(1.0 / self.fps)
  139  
  140          self._release_camera()
  141  
  142      def _open_camera(self):
  143          self._release_camera()
  144          self.cap = cv2.VideoCapture(self.camera_id)
  145          if self.cap.isOpened():
  146              self.cap.set(cv2.CAP_PROP_FRAME_WIDTH, self.width)
  147              self.cap.set(cv2.CAP_PROP_FRAME_HEIGHT, self.height)
  148              self.cap.set(cv2.CAP_PROP_FPS, self.fps)
  149              return True
  150          return False
  151  
  152      def _release_camera(self):
  153          if self.cap is not None:
  154              self.cap.release()
  155              self.cap = None
  156  
  157      def is_connected(self):
  158          return self._connected
  159  
  160      def stop(self):
  161          self._mutex.lock()
  162          self.running = False
  163          self._mutex.unlock()
  164          self.wait(3000)
  165  
  166  
  167  class DetectionThread(QThread):
  168      """检测推理线程"""
  169  
  170      result_ready = pyqtSignal(dict, np.ndarray)
  171      error_occurred = pyqtSignal(str)
  172  
  173      def __init__(self, detector):
  174          super().__init__()
  175          self.detector = detector
  176          self._frame = None
  177          self._has_new_frame = False
  178          self.running = False
  179          self._mutex = QMutex()
  180          self._wait_condition = QWaitCondition()
  181  
  182      def set_frame(self, frame):
  183          self._mutex.lock()
  184          self._frame = frame.copy() if frame is not None else None
  185          self._has_new_frame = True
  186          self._mutex.unlock()
  187          self._wait_condition.wakeOne()
  188  
  189      def run(self):
  190          self.running = True
  191  
  192          while self.running:
  193              self._mutex.lock()
  194              while not self._has_new_frame and self.running:
  195                  self._wait_condition.wait(self._mutex, 100)
  196  
  197              frame = self._frame
  198              self._frame = None
  199              self._has_new_frame = False
  200              self._mutex.unlock()
  201  
  202              if frame is not None and self.running:
  203                  try:
  204                      results = self.detector.detect(frame)
  205                      vis_frame = self.detector.draw_results(frame, results)
  206                      self.result_ready.emit(results, vis_frame)
  207                  except Exception as e:
  208                      self.error_occurred.emit(str(e))
  209  
  210      def stop(self):
  211          self._mutex.lock()
  212          self.running = False
  213          self._mutex.unlock()
  214          self._wait_condition.wakeAll()
  215          self.wait(3000)
  216  
  217  
  218  class PassRateWidget(QWidget):
  219      """合格率显示组件 - 圆形进度条"""
  220  
  221      def __init__(self, parent=None):
  222          super().__init__(parent)
  223          self._rate = 0.0
  224          self.setMinimumSize(120, 120)
  225  
  226      def set_rate(self, rate):
  227          self._rate = max(0.0, min(100.0, rate))
  228          self.update()
  229  
  230      def paintEvent(self, event):
  231          painter = QPainter(self)
  232          painter.setRenderHint(QPainter.Antialiasing)
  233  
  234          side = min(self.width(), self.height())
  235          painter.setViewport((self.width() - side) // 2, (self.height() - side) // 2, side, side)
  236          painter.setWindow(0, 0, 120, 120)
  237  
  238          pen = QPen(QColor("#e0e0e0"), 8)
  239          painter.setPen(pen)
  240          painter.drawArc(15, 15, 90, 90, 0, 360 * 16)
  241  
  242          if self._rate > 0:
  243              if self._rate >= 90:
  244                  color = QColor("#10B981")
  245              elif self._rate >= 70:
  246                  color = QColor("#F59E0B")
  247              else:
  248                  color = QColor("#EF4444")
  249  
  250              pen = QPen(color, 8)
  251              painter.setPen(pen)
  252              span_angle = int(self._rate / 100.0 * 360 * 16)
  253              painter.drawArc(15, 15, 90, 90, 90 * 16, -span_angle)
  254  
  255          painter.setPen(QColor("#333333"))
  256          font = QFont("Arial", 16, QFont.Bold)
  257          painter.setFont(font)
  258          painter.drawText(QRect(0, 0, 120, 120), Qt.AlignCenter, f"{self._rate:.1f}%")
  259  
  260          painter.end()
  261  
  262  
  263  class DefectBarWidget(QWidget):
  264      """缺陷类别分布柱状图组件"""
  265  
  266      def __init__(self, parent=None):
  267          super().__init__(parent)
  268          self._data = {}
  269          self.setMinimumHeight(160)
  270  
  271      def set_data(self, data):
  272          self._data = data
  273          self.update()
  274  
  275      def paintEvent(self, event):
  276          if not self._data:
  277              return
  278  
  279          painter = QPainter(self)
  280          painter.setRenderHint(QPainter.Antialiasing)
  281  
  282          w = self.width()
  283          h = self.height()
  284          margin_left = 40
  285          margin_bottom = 30
  286          margin_top = 10
  287          margin_right = 10
  288  
  289          chart_w = w - margin_left - margin_right
  290          chart_h = h - margin_bottom - margin_top
  291  
  292          categories = list(self._data.keys())
  293          values = list(self._data.values())
  294          max_val = max(values) if values else 1
  295  
  296          bar_width = max(10, min(40, chart_w // len(categories) - 10))
  297          spacing = (chart_w - bar_width * len(categories)) / (len(categories) + 1)
  298  
  299          colors = [
  300              QColor("#EF4444"), QColor("#F59E0B"), QColor("#3B82F6"),
  301              QColor("#10B981"), QColor("#8B5CF6"), QColor("#EC4899"),
  302              QColor("#14B8A6"), QColor("#F97316"),
  303          ]
  304  
  305          painter.setPen(QPen(QColor("#999999"), 1))
  306          painter.drawLine(margin_left, margin_top, margin_left, h - margin_bottom)
  307          painter.drawLine(margin_left, h - margin_bottom, w - margin_right, h - margin_bottom)
  308  
  309          font = QFont("Arial", 8)
  310          painter.setFont(font)
  311  
  312          for i, (cat, val) in enumerate(self._data.items()):
  313              x = margin_left + spacing + i * (bar_width + spacing)
  314              bar_h = (val / max_val) * chart_h if max_val > 0 else 0
  315              y = h - margin_bottom - bar_h
  316  
  317              color = colors[i % len(colors)]
  318              painter.setBrush(QBrush(color))
  319              painter.setPen(Qt.NoPen)
  320              painter.drawRect(int(x), int(y), bar_width, int(bar_h))
  321  
  322              painter.setPen(QColor("#333333"))
  323              painter.drawText(int(x - 5), h - margin_bottom + 15, bar_width + 10, 15,
  324                               Qt.AlignCenter, cat[:4])
  325              painter.drawText(int(x - 5), int(y) - 15, bar_width + 10, 15,
  326                               Qt.AlignCenter, str(val))
  327  
  328          painter.end()
  329  
  330  
  331  class LoginDialog(QDialog):
  332      """登录对话框"""
  333  
  334      def __init__(self, auth_manager, parent=None):
  335          super().__init__(parent)
  336          self.auth_manager = auth_manager
  337          self.setWindowTitle("系统登录")
  338          self.setFixedSize(350, 220)
  339  
  340          layout = QFormLayout(self)
  341  
  342          self.username_edit = QLineEdit()
  343          self.username_edit.setPlaceholderText("请输入用户名")
  344          layout.addRow("用户名:", self.username_edit)
  345  
  346          self.password_edit = QLineEdit()
  347          self.password_edit.setPlaceholderText("请输入密码")
  348          self.password_edit.setEchoMode(QLineEdit.Password)
  349          layout.addRow("密码:", self.password_edit)
  350  
  351          self.operator_id_edit = QLineEdit()
  352          self.operator_id_edit.setPlaceholderText("请输入工号")
  353          layout.addRow("工号:", self.operator_id_edit)
  354  
  355          buttons = QDialogButtonBox(QDialogButtonBox.Ok | QDialogButtonBox.Cancel)
  356          buttons.accepted.connect(self._try_login)
  357          buttons.rejected.connect(self.reject)
  358          layout.addRow(buttons)
  359  
  360          self.logged_in = False
  361  
  362      def _try_login(self):
  363          username = self.username_edit.text().strip()
  364          password = self.password_edit.text().strip()
  365          operator_id = self.operator_id_edit.text().strip()
  366  
  367          if not username or not password:
  368              QMessageBox.warning(self, "提示", "请输入用户名和密码")
  369              return
  370  
  371          if self.auth_manager.login(username, password):
  372              self.logged_in = True
  373              # 设置操作员工号
  374              if operator_id:
  375                  self.auth_manager.current_user.employee_id = operator_id
  376              self.accept()
  377          else:
  378              QMessageBox.warning(self, "登录失败", "用户名或密码错误")
  379  
  380  
  381  class LineSwitchDialog(QDialog):
  382      """一键换线对话框"""
  383  
  384      def __init__(self, production_manager, parent=None):
  385          super().__init__(parent)
  386          self.pm = production_manager
  387          self.setWindowTitle("一键换线")
  388          self.setFixedSize(400, 350)
  389  
  390          layout = QVBoxLayout(self)
  391  
  392          # 当前产线
  393          current_label = QLabel(f"当前产线: {self.pm.current_profile_name}")
  394          current_label.setStyleSheet("font-size: 14px; font-weight: bold; padding: 5px;")
  395          layout.addWidget(current_label)
  396  
  397          # 产线列表
  398          layout.addWidget(QLabel("选择产线:"))
  399          self.profile_list = QListWidget()
  400          for name in self.pm.list_profiles():
  401              self.profile_list.addItem(name)
  402          self.profile_list.itemDoubleClicked.connect(self._switch)
  403          layout.addWidget(self.profile_list)
  404  
  405          # 按钮行
  406          btn_layout = QHBoxLayout()
  407  
  408          switch_btn = QPushButton("切换")
  409          switch_btn.setStyleSheet("background-color: #3B82F6; color: white; padding: 8px;")
  410          switch_btn.clicked.connect(self._switch)
  411          btn_layout.addWidget(switch_btn)
  412  
  413          new_btn = QPushButton("新建产线")
  414          new_btn.setStyleSheet("padding: 8px;")
  415          new_btn.clicked.connect(self._new_profile)
  416          btn_layout.addWidget(new_btn)
  417  
  418          delete_btn = QPushButton("删除")
  419          delete_btn.setStyleSheet("background-color: #EF4444; color: white; padding: 8px;")
  420          delete_btn.clicked.connect(self._delete_profile)
  421          btn_layout.addWidget(delete_btn)
  422  
  423          close_btn = QPushButton("关闭")
  424          close_btn.setStyleSheet("padding: 8px;")
  425          close_btn.clicked.connect(self.reject)
  426          btn_layout.addWidget(close_btn)
  427  
  428          layout.addLayout(btn_layout)
  429  
  430          self.switched = False
  431  
  432      def _switch(self):
  433          item = self.profile_list.currentItem()
  434          if not item:
  435              QMessageBox.warning(self, "提示", "请先选择一个产线配置")
  436              return
  437  
  438          name = item.text()
  439          reply = QMessageBox.question(self, "确认换线",
  440                                       f"确认切换到产线 [{name}]？\n系统将自动更新模型和参数。",
  441                                       QMessageBox.Yes | QMessageBox.No)
  442          if reply == QMessageBox.Yes:
  443              if self.pm.switch_line(name):
  444                  QMessageBox.information(self, "换线成功", f"已切换到产线: {name}")
  445                  self.switched = True
  446                  self.accept()
  447              else:
  448                  QMessageBox.critical(self, "换线失败", f"切换产线 [{name}] 失败")
  449  
  450      def _new_profile(self):
  451          name, ok = QInputDialog.getText(self, "新建产线", "产线名称:")
  452          if ok and name.strip():
  453              profile = self.pm.create_profile_from_current(name.strip())
  454              self.pm.save_profile(profile)
  455              self.profile_list.addItem(name.strip())
  456  
  457      def _delete_profile(self):
  458          item = self.profile_list.currentItem()
  459          if not item:
  460              return
  461          name = item.text()
  462          reply = QMessageBox.question(self, "确认删除", f"确认删除产线配置 [{name}]？",
  463                                       QMessageBox.Yes | QMessageBox.No)
  464          if reply == QMessageBox.Yes:
  465              self.pm.delete_profile(name)
  466              self.profile_list.takeItem(self.profile_list.row(item))
  467  
  468  
  469  class SuppressionDialog(QDialog):
  470      """误报屏蔽管理对话框"""
  471  
  472      def __init__(self, suppressor, parent=None):
  473          super().__init__(parent)
  474          self.suppressor = suppressor
  475          self.setWindowTitle("误报屏蔽管理")
  476          self.setFixedSize(500, 400)
  477  
  478          layout = QVBoxLayout(self)
  479  
  480          # 规则列表
  481          self.rule_table = QTableWidget()
  482          self.rule_table.setColumnCount(5)
  483          self.rule_table.setHorizontalHeaderLabels(["名称", "类型", "区域", "启用", "命中次数"])
  484          self.rule_table.horizontalHeader().setSectionResizeMode(QHeaderView.Stretch)
  485          self._refresh_rules()
  486          layout.addWidget(self.rule_table)
  487  
  488          # 按钮行
  489          btn_layout = QHBoxLayout()
  490  
  491          add_btn = QPushButton("添加矩形区域")
  492          add_btn.setStyleSheet("background-color: #10B981; color: white; padding: 8px;")
  493          add_btn.clicked.connect(self._add_rect_rule)
  494          btn_layout.addWidget(add_btn)
  495  
  496          toggle_btn = QPushButton("启用/禁用")
  497          toggle_btn.setStyleSheet("padding: 8px;")
  498          toggle_btn.clicked.connect(self._toggle_rule)
  499          btn_layout.addWidget(toggle_btn)
  500  
  501          delete_btn = QPushButton("删除")
  502          delete_btn.setStyleSheet("background-color: #EF4444; color: white; padding: 8px;")
  503          delete_btn.clicked.connect(self._delete_rule)
  504          btn_layout.addWidget(delete_btn)
  505  
  506          close_btn = QPushButton("关闭")
  507          close_btn.setStyleSheet("padding: 8px;")
  508          close_btn.clicked.connect(self.accept)
  509          btn_layout.addWidget(close_btn)
  510  
  511          layout.addLayout(btn_layout)
  512  
  513      def _refresh_rules(self):
  514          rules = self.suppressor.list_rules()
  515          self.rule_table.setRowCount(len(rules))
  516          for i, rule in enumerate(rules):
  517              self.rule_table.setItem(i, 0, QTableWidgetItem(rule.name))
  518              self.rule_table.setItem(i, 1, QTableWidgetItem(rule.rule_type))
  519              if rule.rule_type == "rect":
  520                  r = rule.region
  521                  region_str = f"({r.get('x1',0)},{r.get('y1',0)})-({r.get('x2',0)},{r.get('y2',0)})"
  522              else:
  523                  region_str = f"{len(rule.region.get('points',[]))}点"
  524              self.rule_table.setItem(i, 2, QTableWidgetItem(region_str))
  525              self.rule_table.setItem(i, 3, QTableWidgetItem("是" if rule.enabled else "否"))
  526              self.rule_table.setItem(i, 4, QTableWidgetItem(str(rule.hit_count)))
  527  
  528      def _add_rect_rule(self):
  529          from PyQt5.QtWidgets import QInputDialog
  530          name, ok = QInputDialog.getText(self, "添加屏蔽区域", "规则名称:")
  531          if not ok or not name.strip():
  532              return
  533  
  534          dialog = QDialog(self)
  535          dialog.setWindowTitle("设置屏蔽区域坐标")
  536          dialog.setFixedSize(300, 200)
  537          form = QFormLayout(dialog)
  538  
  539          x1_spin = QSpinBox(); x1_spin.setRange(0, 9999); x1_spin.setValue(0)
  540          y1_spin = QSpinBox(); y1_spin.setRange(0, 9999); y1_spin.setValue(0)
  541          x2_spin = QSpinBox(); x2_spin.setRange(0, 9999); x2_spin.setValue(100)
  542          y2_spin = QSpinBox(); y2_spin.setRange(0, 9999); y2_spin.setValue(100)
  543  
  544          form.addRow("左上X:", x1_spin)
  545          form.addRow("左上Y:", y1_spin)
  546          form.addRow("右下X:", x2_spin)
  547          form.addRow("右下Y:", y2_spin)
  548  
  549          buttons = QDialogButtonBox(QDialogButtonBox.Ok | QDialogButtonBox.Cancel)
  550          buttons.accepted.connect(dialog.accept)
  551          buttons.rejected.connect(dialog.reject)
  552          form.addRow(buttons)
  553  
  554          if dialog.exec_() == QDialog.Accepted:
  555              self.suppressor.add_rect_rule(
  556                  name=name.strip(),
  557                  x1=x1_spin.value(), y1=y1_spin.value(),
  558                  x2=x2_spin.value(), y2=y2_spin.value(),
  559              )
  560              self._refresh_rules()
  561  
  562      def _toggle_rule(self):
  563          row = self.rule_table.currentRow()
  564          if row < 0:
  565              return
  566          name = self.rule_table.item(row, 0).text()
  567          self.suppressor.toggle_rule(name)
  568          self._refresh_rules()
  569  
  570      def _delete_rule(self):
  571          row = self.rule_table.currentRow()
  572          if row < 0:
  573              return
  574          name = self.rule_table.item(row, 0).text()
  575          self.suppressor.remove_rule(name)
  576          self._refresh_rules()
  577  
  578  
  579  class BatchReviewDialog(QDialog):
  580      """批量复核对话框"""
  581  
  582      def __init__(self, db, auth_manager, parent=None):
  583          super().__init__(parent)
  584          self.db = db
  585          self.auth = auth_manager
  586          self.setWindowTitle("批量复核")
  587          self.setFixedSize(800, 500)
  588  
  589          layout = QVBoxLayout(self)
  590  
  591          # 未复核记录列表
  592          layout.addWidget(QLabel("未复核的NG记录:"))
  593          self.record_table = QTableWidget()
  594          self.record_table.setColumnCount(7)
  595          self.record_table.setHorizontalHeaderLabels(
  596              ["选择", "ID", "时间", "SN码", "操作员", "缺陷数", "设备"])
  597          self.record_table.horizontalHeader().setSectionResizeMode(QHeaderView.Stretch)
  598          layout.addWidget(self.record_table)
  599  
  600          self._load_records()
  601  
  602          # 按钮行
  603          btn_layout = QHBoxLayout()
  604  
  605          select_all_btn = QPushButton("全选")
  606          select_all_btn.clicked.connect(self._select_all)
  607          btn_layout.addWidget(select_all_btn)
  608  
  609          confirm_btn = QPushButton("批量确认（正确检出）")
  610          confirm_btn.setStyleSheet("background-color: #10B981; color: white; padding: 8px;")
  611          confirm_btn.clicked.connect(lambda: self._batch_review("confirmed"))
  612          btn_layout.addWidget(confirm_btn)
  613  
  614          false_alarm_btn = QPushButton("批量标记（误报）")
  615          false_alarm_btn.setStyleSheet("background-color: #F59E0B; color: white; padding: 8px;")
  616          false_alarm_btn.clicked.connect(lambda: self._batch_review("false_alarm"))
  617          btn_layout.addWidget(false_alarm_btn)
  618  
  619          close_btn = QPushButton("关闭")
  620          close_btn.clicked.connect(self.accept)
  621          btn_layout.addWidget(close_btn)
  622  
  623          layout.addLayout(btn_layout)
  624  
  625      def _load_records(self):
  626          records = self.db.get_unreviewed(limit=50)
  627          self.record_table.setRowCount(len(records))
  628          self._checkboxes = []
  629  
  630          for i, rec in enumerate(records):
  631              cb = QCheckBox()
  632              self._checkboxes.append(cb)
  633              cb_widget = QWidget()
  634              cb_layout = QHBoxLayout(cb_widget)
  635              cb_layout.addWidget(cb)
  636              cb_layout.setAlignment(Qt.AlignCenter)
  637              cb_layout.setContentsMargins(0, 0, 0, 0)
  638  
  639              self.record_table.setCellWidget(i, 0, cb_widget)
  640              self.record_table.setItem(i, 1, QTableWidgetItem(str(rec.get('id', ''))))
  641              self.record_table.setItem(i, 2, QTableWidgetItem(rec.get('timestamp', '')[:19]))
  642              self.record_table.setItem(i, 3, QTableWidgetItem(rec.get('product_sn', '')))
  643              self.record_table.setItem(i, 4, QTableWidgetItem(rec.get('operator_id', '')))
  644              self.record_table.setItem(i, 5, QTableWidgetItem(str(rec.get('num_defects', 0))))
  645              self.record_table.setItem(i, 6, QTableWidgetItem(rec.get('equipment_id', '')))
  646  
  647      def _select_all(self):
  648          for cb in self._checkboxes:
  649              cb.setChecked(not cb.isChecked())
  650  
  651      def _batch_review(self, result):
  652          ids = []
  653          for i, cb in enumerate(self._checkboxes):
  654              if cb.isChecked():
  655                  id_item = self.record_table.item(i, 1)
  656                  if id_item:
  657                      ids.append(int(id_item.text()))
  658  
  659          if not ids:
  660              QMessageBox.warning(self, "提示", "请先选择要复核的记录")
  661              return
  662  
  663          reviewer = self.auth.current_user.username if self.auth.current_user else "unknown"
  664          self.db.batch_review(ids, result, reviewer)
  665          QMessageBox.information(self, "完成", f"已复核 {len(ids)} 条记录")
  666          self._load_records()
  667  
  668  
  669  class MainWindow(QMainWindow):
  670      """主窗口"""
  671  
  672      def __init__(self, config=None, detector=None, controller=None):
  673          super().__init__()
  674  
  675          self.config = config or get_config()
  676          self.detector = detector
  677          self.controller = controller
  678  
  679          # 初始化子系统
  680          self.production_manager = ProductionManager(
  681              profiles_dir=getattr(self.config, 'production', None) and self.config.production.profiles_dir or None
  682          )
  683          self.suppressor = FalseAlarmSuppressor(
  684              rules_dir=getattr(self.config, 'suppression', None) and self.config.suppression.rules_dir or None
  685          )
  686          self.trace_db = TraceabilityDB(
  687              db_path=getattr(self.config, 'traceability', None) and self.config.traceability.db_path or None
  688          )
  689          self.alarm_manager = AlarmManager(controller=self.controller)
  690          self.equipment_monitor = EquipmentMonitor(config=self.config)
  691          self.auth_manager = AuthManager(
  692              db_path=getattr(self.config, 'auth', None) and self.config.auth.db_path or None
  693          )
  694          self.report_generator = ProductionReportGenerator(self.trace_db)
  695  
  696          # 状态
  697          self.is_running = False
  698          self.is_emergency_stopped = False
  699          self.is_expert_mode = self.config.gui.expert_mode
  700          self.current_frame = None
  701          self.detection_results = None
  702  
  703          # 当前模式
  704          default_mode = getattr(self.config.gui, 'default_mode', 'production')
  705          self.production_manager.set_mode(default_mode)
  706  
  707          # 历史记录
  708          self.history = deque(maxlen=self.config.system.max_history)
  709          self.expert_scores = []
  710          self.total_images = 0
  711          self.correct_images = 0
  712  
  713          # 统计数据
  714          self._defect_counter = Counter()
  715          self._total_detections = 0
  716          self._total_pass = 0
  717          self._total_fail = 0
  718          self._inference_times = deque(maxlen=100)
  719  
  720          # SN码输入
  721          self._current_sn = ""
  722  
  723          # 初始化UI
  724          self.init_ui()
  725          self.init_timers()
  726  
  727          # 注册换线回调
  728          self.production_manager.on_line_switched(self._on_line_switched)
  729          self.production_manager.on_mode_changed(self._on_mode_changed)
  730  
  731          # 注册报警回调
  732          self.alarm_manager.on_alarm(self._on_alarm_triggered)
  733  
  734          logger.info("GUI初始化完成")
  735  
  736      def init_ui(self):
  737          """初始化用户界面"""
  738          self.setWindowTitle(self.config.gui.window_title)
  739          self.setGeometry(100, 100, self.config.gui.window_width, self.config.gui.window_height)
  740  
  741          # 中央部件
  742          central_widget = QWidget()
  743          self.setCentralWidget(central_widget)
  744  
  745          # 主布局
  746          main_layout = QHBoxLayout(central_widget)
  747  
  748          # 分割器
  749          splitter = QSplitter(Qt.Horizontal)
  750          main_layout.addWidget(splitter)
  751  
  752          # ============== 左侧：视频显示 ==============
  753          left_panel = QWidget()
  754          left_layout = QVBoxLayout(left_panel)
  755  
  756          # 顶部工具栏：模式切换 + 换线 + SN码
  757          toolbar_layout = QHBoxLayout()
  758  
  759          self.mode_btn = QPushButton("产线模式")
  760          self.mode_btn.setStyleSheet("font-size: 12px; padding: 6px; background-color: #3B82F6; color: white;")
  761          self.mode_btn.clicked.connect(self.toggle_mode)
  762          toolbar_layout.addWidget(self.mode_btn)
  763  
  764          self.line_switch_btn = QPushButton("一键换线")
  765          self.line_switch_btn.setStyleSheet("font-size: 12px; padding: 6px; background-color: #8B5CF6; color: white;")
  766          self.line_switch_btn.clicked.connect(self.show_line_switch)
  767          toolbar_layout.addWidget(self.line_switch_btn)
  768  
  769          self.line_label = QLabel(f"产线: {self.production_manager.current_profile_name}")
  770          self.line_label.setStyleSheet("font-size: 12px; padding: 3px;")
  771          toolbar_layout.addWidget(self.line_label)
  772  
  773          toolbar_layout.addWidget(QLabel("SN:"))
  774          self.sn_input = QLineEdit()
  775          self.sn_input.setPlaceholderText("扫码/输入SN码")
  776          self.sn_input.setMaximumWidth(160)
  777          self.sn_input.returnPressed.connect(self._on_sn_entered)
  778          toolbar_layout.addWidget(self.sn_input)
  779  
  780          self.suppression_btn = QPushButton("屏蔽区域")
  781          self.suppression_btn.setStyleSheet("font-size: 12px; padding: 6px;")
  782          self.suppression_btn.clicked.connect(self.show_suppression_dialog)
  783          toolbar_layout.addWidget(self.suppression_btn)
  784  
  785          toolbar_layout.addStretch()
  786  
  787          left_layout.addLayout(toolbar_layout)
  788  
  789          # 视频显示标签
  790          self.video_label = QLabel("等待启动...")
  791          self.video_label.setAlignment(Qt.AlignCenter)
  792          self.video_label.setMinimumSize(800, 600)
  793          self.video_label.setStyleSheet("background-color: #1a1a2e; color: white; font-size: 18px;")
  794          left_layout.addWidget(self.video_label)
  795  
  796          # 控制按钮行1
  797          btn_layout = QHBoxLayout()
  798  
  799          self.start_btn = QPushButton("▶ 启动检测")
  800          self.start_btn.setStyleSheet("font-size: 14px; padding: 10px; background-color: #10B981; color: white;")
  801          self.start_btn.clicked.connect(self.toggle_detection)
  802          btn_layout.addWidget(self.start_btn)
  803  
  804          self.capture_btn = QPushButton("截图")
  805          self.capture_btn.setStyleSheet("font-size: 14px; padding: 10px;")
  806          self.capture_btn.clicked.connect(self.capture_image)
  807          self.capture_btn.setEnabled(False)
  808          btn_layout.addWidget(self.capture_btn)
  809  
  810          self.load_btn = QPushButton("加载图片")
  811          self.load_btn.setStyleSheet("font-size: 14px; padding: 10px;")
  812          self.load_btn.clicked.connect(self.load_image)
  813          btn_layout.addWidget(self.load_btn)
  814  
  815          left_layout.addLayout(btn_layout)
  816  
  817          # 控制按钮行2：急停 + 功能按钮
  818          btn_layout2 = QHBoxLayout()
  819  
  820          self.emergency_stop_btn = QPushButton("⛔ 急 停")
  821          self.emergency_stop_btn.setStyleSheet(
  822              "font-size: 20px; font-weight: bold; padding: 18px; "
  823              "background-color: #DC2626; color: white; border: 3px solid #991B1B; "
  824              "border-radius: 8px; min-height: 50px;"
  825          )
  826          self.emergency_stop_btn.clicked.connect(self.emergency_stop)
  827          btn_layout2.addWidget(self.emergency_stop_btn)
  828  
  829          self.export_csv_btn = QPushButton("导出报表")
  830          self.export_csv_btn.setStyleSheet("font-size: 14px; padding: 10px; background-color: #3B82F6; color: white;")
  831          self.export_csv_btn.clicked.connect(self.export_csv)
  832          btn_layout2.addWidget(self.export_csv_btn)
  833  
  834          self.batch_review_btn = QPushButton("批量复核")
  835          self.batch_review_btn.setStyleSheet("font-size: 14px; padding: 10px;")
  836          self.batch_review_btn.clicked.connect(self.show_batch_review)
  837          btn_layout2.addWidget(self.batch_review_btn)
  838  
  839          self.production_report_btn = QPushButton("生产报表")
  840          self.production_report_btn.setStyleSheet("font-size: 14px; padding: 10px;")
  841          self.production_report_btn.clicked.connect(self.show_production_report)
  842          btn_layout2.addWidget(self.production_report_btn)
  843  
  844          left_layout.addLayout(btn_layout2)
  845  
  846          # 状态栏
  847          status_layout = QHBoxLayout()
  848          self.status_label = QLabel("状态: 就绪")
  849          self.status_label.setStyleSheet("font-size: 12px; padding: 5px; background-color: #f0f0f0;")
  850          status_layout.addWidget(self.status_label)
  851  
  852          self.camera_status_label = QLabel("摄像头: 未连接")
  853          self.camera_status_label.setStyleSheet("font-size: 12px; padding: 3px; color: #999;")
  854          status_layout.addWidget(self.camera_status_label)
  855  
  856          self.health_label = QLabel("设备: 正常")
  857          self.health_label.setStyleSheet("font-size: 12px; padding: 3px; color: #10B981;")
  858          status_layout.addWidget(self.health_label)
  859  
  860          self.alarm_label = QLabel("")
  861          self.alarm_label.setStyleSheet("font-size: 12px; padding: 3px; color: #EF4444; font-weight: bold;")
  862          status_layout.addWidget(self.alarm_label)
  863  
  864          left_layout.addLayout(status_layout)
  865  
  866          splitter.addWidget(left_panel)
  867  
  868          # ============== 右侧：信息面板 ==============
  869          right_panel = QWidget()
  870          right_layout = QVBoxLayout(right_panel)
  871          right_panel.setMaximumWidth(520)
  872  
  873          # 标签页
  874          self.tab_widget = QTabWidget()
  875  
  876          # --- 检测结果页 ---
  877          result_tab = QWidget()
  878          result_layout = QVBoxLayout(result_tab)
  879  
  880          self.info_group = QGroupBox("检测信息")
  881          info_layout = QGridLayout()
  882  
  883          self.fps_label = QLabel("FPS: --")
  884          self.time_label = QLabel("推理时间: -- ms")
  885          self.count_label = QLabel("检测数量: --")
  886          self.conf_label = QLabel("平均置信度: --")
  887  
  888          info_layout.addWidget(QLabel("帧率:"), 0, 0)
  889          info_layout.addWidget(self.fps_label, 0, 1)
  890          info_layout.addWidget(QLabel("推理时间:"), 1, 0)
  891          info_layout.addWidget(self.time_label, 1, 1)
  892          info_layout.addWidget(QLabel("检测数量:"), 2, 0)
  893          info_layout.addWidget(self.count_label, 2, 1)
  894          info_layout.addWidget(QLabel("平均置信度:"), 3, 0)
  895          info_layout.addWidget(self.conf_label, 3, 1)
  896  
  897          self.info_group.setLayout(info_layout)
  898          result_layout.addWidget(self.info_group)
  899  
  900          self.result_table = QTableWidget()
  901          self.result_table.setColumnCount(4)
  902          self.result_table.setHorizontalHeaderLabels(["类别", "置信度", "位置", "操作"])
  903          self.result_table.horizontalHeader().setSectionResizeMode(QHeaderView.Stretch)
  904          result_layout.addWidget(self.result_table)
  905  
  906          self.tab_widget.addTab(result_tab, "检测结果")
  907  
  908          # --- 专家评分页 ---
  909          if self.is_expert_mode:
  910              expert_tab = QWidget()
  911              expert_layout = QVBoxLayout(expert_tab)
  912  
  913              self.score_label = QLabel("评分统计: 0/0 (0%)")
  914              self.score_label.setStyleSheet("font-size: 16px; font-weight: bold; padding: 10px;")
  915              expert_layout.addWidget(self.score_label)
  916  
  917              score_btn_layout = QHBoxLayout()
  918  
  919              self.correct_btn = QPushButton("正确")
  920              self.correct_btn.setStyleSheet("font-size: 14px; padding: 15px; background-color: #10B981; color: white;")
  921              self.correct_btn.clicked.connect(lambda: self.score_detection(True))
  922              score_btn_layout.addWidget(self.correct_btn)
  923  
  924              self.wrong_btn = QPushButton("错误")
  925              self.wrong_btn.setStyleSheet("font-size: 14px; padding: 15px; background-color: #EF4444; color: white;")
  926              self.wrong_btn.clicked.connect(lambda: self.score_detection(False))
  927              score_btn_layout.addWidget(self.wrong_btn)
  928  
  929              expert_layout.addLayout(score_btn_layout)
  930  
  931              self.score_table = QTableWidget()
  932              self.score_table.setColumnCount(4)
  933              self.score_table.setHorizontalHeaderLabels(["序号", "时间", "结果", "正确"])
  934              expert_layout.addWidget(self.score_table)
  935  
  936              self.tab_widget.addTab(expert_tab, "专家评分")
  937  
  938          # --- 统计仪表盘页 ---
  939          stats_tab = QWidget()
  940          stats_layout = QVBoxLayout(stats_tab)
  941  
  942          pass_rate_group = QGroupBox("合格率")
  943          pass_rate_layout = QHBoxLayout()
  944  
  945          self.pass_rate_widget = PassRateWidget()
  946          pass_rate_layout.addWidget(self.pass_rate_widget)
  947  
  948          self.pass_rate_detail_label = QLabel("合格: 0 | 缺陷: 0 | 总计: 0")
  949          self.pass_rate_detail_label.setStyleSheet("font-size: 12px; padding: 5px;")
  950          pass_rate_layout.addWidget(self.pass_rate_detail_label)
  951  
  952          pass_rate_group.setLayout(pass_rate_layout)
  953          stats_layout.addWidget(pass_rate_group)
  954  
  955          defect_dist_group = QGroupBox("缺陷类别分布")
  956          defect_dist_layout = QVBoxLayout()
  957  
  958          self.defect_bar_widget = DefectBarWidget()
  959          defect_dist_layout.addWidget(self.defect_bar_widget)
  960  
  961          defect_dist_group.setLayout(defect_dist_layout)
  962          stats_layout.addWidget(defect_dist_group)
  963  
  964          perf_group = QGroupBox("性能统计")
  965          perf_layout = QGridLayout()
  966  
  967          self.avg_inference_label = QLabel("平均推理耗时: -- ms")
  968          self.max_inference_label = QLabel("最大推理耗时: -- ms")
  969          self.min_inference_label = QLabel("最小推理耗时: -- ms")
  970          self.total_detected_label = QLabel("累计检测帧数: 0")
  971  
  972          perf_layout.addWidget(self.avg_inference_label, 0, 0)
  973          perf_layout.addWidget(self.max_inference_label, 0, 1)
  974          perf_layout.addWidget(self.min_inference_label, 1, 0)
  975          perf_layout.addWidget(self.total_detected_label, 1, 1)
  976  
  977          perf_group.setLayout(perf_layout)
  978          stats_layout.addWidget(perf_group)
  979  
  980          # 设备健康度
  981          health_group = QGroupBox("设备健康度")
  982          health_layout = QGridLayout()
  983  
  984          self.light_health_label = QLabel("光源: 正常")
  985          self.camera_health_label = QLabel("摄像头: 正常")
  986          self.inference_health_label = QLabel("推理: 正常")
  987          self.brightness_label = QLabel("亮度: --")
  988  
  989          health_layout.addWidget(self.light_health_label, 0, 0)
  990          health_layout.addWidget(self.camera_health_label, 0, 1)
  991          health_layout.addWidget(self.inference_health_label, 1, 0)
  992          health_layout.addWidget(self.brightness_label, 1, 1)
  993  
  994          health_group.setLayout(health_layout)
  995          stats_layout.addWidget(health_group)
  996  
  997          stats_layout.addStretch()
  998          self.tab_widget.addTab(stats_tab, "统计仪表盘")
  999  
 1000          # --- 历史记录页 ---
 1001          history_tab = QWidget()
 1002          history_layout = QVBoxLayout(history_tab)
 1003  
 1004          # 追溯查询
 1005          query_layout = QHBoxLayout()
 1006          query_layout.addWidget(QLabel("SN码:"))
 1007          self.query_sn_edit = QLineEdit()
 1008          self.query_sn_edit.setPlaceholderText("输入SN码查询")
 1009          query_layout.addWidget(self.query_sn_edit)
 1010          query_btn = QPushButton("查询")
 1011          query_btn.clicked.connect(self._query_by_sn)
 1012          query_layout.addWidget(query_btn)
 1013          history_layout.addLayout(query_layout)
 1014  
 1015          self.history_text = QTextEdit()
 1016          self.history_text.setReadOnly(True)
 1017          history_layout.addWidget(self.history_text)
 1018  
 1019          self.tab_widget.addTab(history_tab, "历史记录")
 1020  
 1021          right_layout.addWidget(self.tab_widget)
 1022  
 1023          # 系统日志
 1024          self.log_text = QTextEdit()
 1025          self.log_text.setReadOnly(True)
 1026          self.log_text.setMaximumHeight(150)
 1027          self.log_text.setStyleSheet("background-color: #f8f9fa; font-family: monospace; font-size: 10px;")
 1028          right_layout.addWidget(QLabel("系统日志:"))
 1029          right_layout.addWidget(self.log_text)
 1030  
 1031          splitter.addWidget(right_panel)
 1032          splitter.setSizes([900, 520])
 1033  
 1034      def init_timers(self):
 1035          """初始化定时器"""
 1036          self.refresh_timer = QTimer()
 1037          self.refresh_timer.timeout.connect(self.update_display)
 1038          self.refresh_timer.start(self.config.gui.refresh_interval)
 1039  
 1040          self.status_timer = QTimer()
 1041          self.status_timer.timeout.connect(self.update_status)
 1042          self.status_timer.start(1000)
 1043  
 1044          # 设备健康度更新
 1045          self.health_timer = QTimer()
 1046          self.health_timer.timeout.connect(self.update_health_display)
 1047          self.health_timer.start(5000)
 1048  
 1049      # ==================== 模式切换 ====================
 1050  
 1051      def toggle_mode(self):
 1052          """切换产线/调试模式"""
 1053          if self.production_manager.is_production_mode:
 1054              self.production_manager.set_mode(ProductionManager.MODE_DEBUG)
 1055          else:
 1056              self.production_manager.set_mode(ProductionManager.MODE_PRODUCTION)
 1057  
 1058      def _on_mode_changed(self, mode):
 1059          """模式切换回调"""
 1060          if mode == ProductionManager.MODE_PRODUCTION:
 1061              self.mode_btn.setText("产线模式")
 1062              self.mode_btn.setStyleSheet("font-size: 12px; padding: 6px; background-color: #3B82F6; color: white;")
 1063              # 产线模式：隐藏调试信息
 1064              if hasattr(self, 'info_group'):
 1065                  self.info_group.setTitle("检测信息")
 1066          else:
 1067              self.mode_btn.setText("调试模式")
 1068              self.mode_btn.setStyleSheet("font-size: 12px; padding: 6px; background-color: #F59E0B; color: white;")
 1069              if hasattr(self, 'info_group'):
 1070                  self.info_group.setTitle("检测信息 (调试)")
 1071  
 1072      # ==================== 一键换线 ====================
 1073  
 1074      def show_line_switch(self):
 1075          """显示一键换线对话框"""
 1076          dialog = LineSwitchDialog(self.production_manager, self)
 1077          dialog.exec_()
 1078          self.line_label.setText(f"产线: {self.production_manager.current_profile_name}")
 1079  
 1080      def _on_line_switched(self, profile):
 1081          """换线回调 - 更新检测器参数"""
 1082          if self.detector:
 1083              try:
 1084                  self.detector.conf_threshold = profile.conf_threshold
 1085                  self.detector.iou_threshold = profile.iou_threshold
 1086                  logger.info(f"检测器参数已更新: conf={profile.conf_threshold}, iou={profile.iou_threshold}")
 1087              except Exception as e:
 1088                  logger.error(f"更新检测器参数失败: {e}")
 1089  
 1090          # 切换误报屏蔽规则
 1091          self.suppressor.set_profile(profile.name)
 1092  
 1093      # ==================== 误报屏蔽 ====================
 1094  
 1095      def show_suppression_dialog(self):
 1096          """显示误报屏蔽管理对话框"""
 1097          dialog = SuppressionDialog(self.suppressor, self)
 1098          dialog.exec_()
 1099  
 1100      # ==================== SN码 ====================
 1101  
 1102      # SN码最大长度限制
 1103      SN_MAX_LENGTH = 64
 1104      # SN码正则：只允许字母、数字、连字符、下划线
 1105      SN_PATTERN = re.compile(r'^[A-Za-z0-9_-]+$')
 1106  
 1107      def _on_sn_entered(self):
 1108          """SN码输入回调"""
 1109          sn = self.sn_input.text().strip()
 1110          if not sn:
 1111              return
 1112          # 验证SN码格式
 1113          if len(sn) > self.SN_MAX_LENGTH:
 1114              self.log_text.append(f"[WARNING] SN码长度超过限制({self.SN_MAX_LENGTH}字符)")
 1115              sn = sn[:self.SN_MAX_LENGTH]
 1116          if not self.SN_PATTERN.match(sn):
 1117              self.log_text.append(f"[WARNING] SN码包含非法字符，只允许字母、数字、连字符、下划线")
 1118              # 过滤掉非法字符
 1119              sn = ''.join(c for c in sn if c.isalnum() or c in '_-')
 1120              if not sn:
 1121                  return
 1122          self._current_sn = sn
 1123          self.log_text.append(f"[INFO] SN码已设置: {self._current_sn}")
 1124          self.sn_input.clear()
 1125  
 1126      # ==================== 批量复核 ====================
 1127  
 1128      def show_batch_review(self):
 1129          """显示批量复核对话框"""
 1130          dialog = BatchReviewDialog(self.trace_db, self.auth_manager, self)
 1131          dialog.exec_()
 1132  
 1133      # ==================== 生产报表 ====================
 1134  
 1135      def show_production_report(self):
 1136          """显示生产报表"""
 1137          from PyQt5.QtWidgets import QInputDialog
 1138          modes = ["班次报表", "缺陷趋势(7天)", "操作员绩效"]
 1139          mode, ok = QInputDialog.getItem(self, "生产报表", "选择报表类型:", modes, 0, False)
 1140          if not ok:
 1141              return
 1142  
 1143          file_path, _ = QFileDialog.getSaveFileName(
 1144              self, "保存报表",
 1145              f"report_{datetime.now().strftime('%Y%m%d_%H%M%S')}.csv",
 1146              "CSV文件 (*.csv);;JSON文件 (*.json)"
 1147          )
 1148          if not file_path:
 1149              return
 1150  
 1151          try:
 1152              if mode == "班次报表":
 1153                  report = self.report_generator.generate_shift_report()
 1154              elif mode == "缺陷趋势(7天)":
 1155                  report = self.report_generator.generate_defect_trend(days=7)
 1156              else:
 1157                  report = self.report_generator.generate_operator_report()
 1158  
 1159              if file_path.endswith('.json'):
 1160                  self.report_generator.export_json(report, file_path)
 1161              else:
 1162                  self.report_generator.export_csv(report, file_path)
 1163  
 1164              QMessageBox.information(self, "导出成功", f"报表已保存至:\n{file_path}")
 1165          except Exception as e:
 1166              QMessageBox.critical(self, "导出失败", f"报表生成失败:\n{str(e)}")
 1167  
 1168      # ==================== 追溯查询 ====================
 1169  
 1170      def _query_by_sn(self):
 1171          """按SN码查询"""
 1172          sn = self.query_sn_edit.text().strip()
 1173          if not sn:
 1174              return
 1175  
 1176          records = self.trace_db.query_by_sn(sn)
 1177          if not records:
 1178              self.history_text.append(f"未找到SN码 [{sn}] 的记录")
 1179              return
 1180  
 1181          self.history_text.append(f"\n=== SN码查询: {sn} ===")
 1182          for rec in records:
 1183              self.history_text.append(
 1184                  f"  时间: {rec['timestamp'][:19]} | 结果: {rec['result']} | "
 1185                  f"缺陷数: {rec['num_defects']} | 操作员: {rec['operator_id']} | "
 1186                  f"设备: {rec['equipment_id']}"
 1187              )
 1188  
 1189      # ==================== 报警 ====================
 1190  
 1191      def _on_alarm_triggered(self, level, defect_count, message):
 1192          """报警回调 - 更新UI"""
 1193          level_names = {AlarmLevel.INFO: "提示", AlarmLevel.WARNING: "警告", AlarmLevel.CRITICAL: "严重"}
 1194          colors = {AlarmLevel.INFO: "#3B82F6", AlarmLevel.WARNING: "#F59E0B", AlarmLevel.CRITICAL: "#EF4444"}
 1195  
 1196          self.alarm_label.setText(f"报警: [{level_names.get(level, level)}] {message}")
 1197          self.alarm_label.setStyleSheet(
 1198              f"font-size: 12px; padding: 3px; color: {colors.get(level, '#EF4444')}; font-weight: bold;"
 1199          )
 1200  
 1201          # 3秒后清除报警显示
 1202          QTimer.singleShot(3000, lambda: self.alarm_label.setText(""))
 1203  
 1204      # ==================== 设备健康度 ====================
 1205  
 1206      def update_health_display(self):
 1207          """更新设备健康度显示"""
 1208          details = self.equipment_monitor.get_details()
 1209          status_colors = {
 1210              HealthStatus.GOOD: "#10B981",
 1211              HealthStatus.WARNING: "#F59E0B",
 1212              HealthStatus.CRITICAL: "#EF4444",
 1213          }
 1214          status_names = {HealthStatus.GOOD: "正常", HealthStatus.WARNING: "注意", HealthStatus.CRITICAL: "异常"}
 1215  
 1216          light = details.get('light_source', {})
 1217          camera = details.get('camera', {})
 1218          inference = details.get('inference', {})
 1219  
 1220          self.light_health_label.setText(
 1221              f"光源: {status_names.get(light.get('status', 'good'), '正常')}"
 1222          )
 1223          self.light_health_label.setStyleSheet(
 1224              f"color: {status_colors.get(light.get('status', 'good'), '#10B981')}"
 1225          )
 1226  
 1227          self.camera_health_label.setText(
 1228              f"摄像头: {status_names.get(camera.get('status', 'good'), '正常')} ({camera.get('avg_fps', 0):.0f}fps)"
 1229          )
 1230          self.camera_health_label.setStyleSheet(
 1231              f"color: {status_colors.get(camera.get('status', 'good'), '#10B981')}"
 1232          )
 1233  
 1234          self.inference_health_label.setText(
 1235              f"推理: {status_names.get(inference.get('status', 'good'), '正常')} ({inference.get('avg_time_ms', 0):.0f}ms)"
 1236          )
 1237          self.inference_health_label.setStyleSheet(
 1238              f"color: {status_colors.get(inference.get('status', 'good'), '#10B981')}"
 1239          )
 1240  
 1241          self.brightness_label.setText(f"亮度: {light.get('avg_brightness', 0):.0f}")
 1242  
 1243          # 更新状态栏
 1244          overall = self.equipment_monitor.overall_status
 1245          self.health_label.setText(f"设备: {status_names.get(overall, '正常')}")
 1246          self.health_label.setStyleSheet(
 1247              f"font-size: 12px; padding: 3px; color: {status_colors.get(overall, '#10B981')};"
 1248          )
 1249  
 1250      # ==================== 检测控制 ====================
 1251  
 1252      def toggle_detection(self):
 1253          if not self.is_running:
 1254              self.start_detection()
 1255          else:
 1256              self.stop_detection()
 1257  
 1258      def start_detection(self):
 1259          if self.detector is None:
 1260              QMessageBox.warning(self, "警告", "检测器未初始化")
 1261              return
 1262  
 1263          self.is_emergency_stopped = False
 1264          self.emergency_stop_btn.setStyleSheet(
 1265              "font-size: 20px; font-weight: bold; padding: 18px; "
 1266              "background-color: #DC2626; color: white; border: 3px solid #991B1B; "
 1267              "border-radius: 8px; min-height: 50px;"
 1268          )
 1269  
 1270          self.camera_thread = CameraThread(
 1271              camera_id=self.config.hardware.camera.index,
 1272              width=self.config.hardware.camera.width,
 1273              height=self.config.hardware.camera.height,
 1274              fps=self.config.hardware.camera.fps
 1275          )
 1276          self.camera_thread.frame_ready.connect(self.on_frame_ready)
 1277          self.camera_thread.error_occurred.connect(self.on_error)
 1278          self.camera_thread.connection_lost.connect(self.on_camera_lost)
 1279          self.camera_thread.connection_restored.connect(self.on_camera_restored)
 1280          self.camera_thread.start()
 1281  
 1282          self.detection_thread = DetectionThread(self.detector)
 1283          self.detection_thread.result_ready.connect(self.on_detection_result)
 1284          self.detection_thread.error_occurred.connect(self.on_error)
 1285          self.detection_thread.start()
 1286  
 1287          self.is_running = True
 1288          self.start_btn.setText("⏹ 停止检测")
 1289          self.start_btn.setStyleSheet("font-size: 14px; padding: 10px; background-color: #EF4444; color: white;")
 1290          self.capture_btn.setEnabled(True)
 1291  
 1292          logger.info("检测已启动")
 1293  
 1294      def stop_detection(self):
 1295          if hasattr(self, 'camera_thread'):
 1296              self.camera_thread.stop()
 1297          if hasattr(self, 'detection_thread'):
 1298              self.detection_thread.stop()
 1299  
 1300          self.is_running = False
 1301          self.start_btn.setText("▶ 启动检测")
 1302          self.start_btn.setStyleSheet("font-size: 14px; padding: 10px; background-color: #10B981; color: white;")
 1303          self.capture_btn.setEnabled(False)
 1304          self.camera_status_label.setText("摄像头: 未连接")
 1305          self.camera_status_label.setStyleSheet("font-size: 12px; padding: 3px; color: #999;")
 1306  
 1307          logger.info("检测已停止")
 1308  
 1309      def emergency_stop(self):
 1310          if self.is_emergency_stopped:
 1311              self.is_emergency_stopped = False
 1312              self.emergency_stop_btn.setStyleSheet(
 1313                  "font-size: 20px; font-weight: bold; padding: 18px; "
 1314                  "background-color: #DC2626; color: white; border: 3px solid #991B1B; "
 1315                  "border-radius: 8px; min-height: 50px;"
 1316              )
 1317              self.emergency_stop_btn.setText("⛔ 急 停")
 1318              self.log_text.append("[INFO] 急停已解除，可重新启动检测")
 1319              return
 1320  
 1321          self.is_emergency_stopped = True
 1322          self.emergency_stop_btn.setStyleSheet(
 1323              "font-size: 20px; font-weight: bold; padding: 18px; "
 1324              "background-color: #7F1D1D; color: #FCA5A5; border: 3px solid #FCA5A5; "
 1325              "border-radius: 8px; min-height: 50px;"
 1326          )
 1327          self.emergency_stop_btn.setText("⛔ 已急停 - 点击解除")
 1328  
 1329          if self.controller is not None:
 1330              try:
 1331                  self.controller.emergency_stop()
 1332              except Exception as e:
 1333                  logger.error(f"硬件急停执行失败: {e}")
 1334  
 1335          self.stop_detection()
 1336          self.log_text.append("[WARNING] 急停已触发！传送带和舵机已停止！")
 1337  
 1338      # ==================== 检测结果处理 ====================
 1339  
 1340      def on_frame_ready(self, frame):
 1341          self.current_frame = frame
 1342          if hasattr(self, 'detection_thread') and not self.is_emergency_stopped:
 1343              self.detection_thread.set_frame(frame)
 1344  
 1345      def on_detection_result(self, results, vis_frame):
 1346          self.detection_results = results
 1347  
 1348          # 误报屏蔽过滤
 1349          if results.get('detections'):
 1350              valid, suppressed = self.suppressor.filter_detections(results['detections'])
 1351              results['detections'] = valid
 1352              results['num_detections'] = len(valid)
 1353              if suppressed:
 1354                  self.log_text.append(f"[INFO] 屏蔽了 {len(suppressed)} 个误报")
 1355  
 1356          # 设备健康度记录
 1357          self.equipment_monitor.record_frame(vis_frame, results.get('inference_time', 0))
 1358  
 1359          # 声光报警
 1360          num_defects = results.get('num_detections', 0)
 1361          if num_defects > 0:
 1362              level = AlarmLevel.CRITICAL if num_defects >= 3 else AlarmLevel.WARNING
 1363              self.alarm_manager.trigger(level, num_defects, f"检出{num_defects}个缺陷")
 1364  
 1365          # 追溯记录（仅 NG 帧写入数据库，避免 OK 帧高频写入导致 I/O 竞争）
 1366          result_status = "NG" if num_defects > 0 else "OK"
 1367          if num_defects > 0:
 1368              shift = self.production_manager.shift_info
 1369              timestamp = datetime.now().strftime("%Y%m%d_%H%M%S")
 1370              save_dir = Path(self.config.system.save_dir)
 1371              save_dir.mkdir(parents=True, exist_ok=True)
 1372              image_path = str(save_dir / f"gui_raw_{timestamp}.jpg")
 1373              result_image_path = str(save_dir / f"gui_result_{timestamp}.jpg")
 1374              cv2.imwrite(image_path, vis_frame)  # 保存带标注的图
 1375              self.trace_db.add_record(
 1376                  product_sn=self._current_sn,
 1377                  operator_id=shift.get('operator_id', ''),
 1378                  equipment_id=shift.get('equipment_id', ''),
 1379                  line_profile=self.production_manager.current_profile_name,
 1380                  result=result_status,
 1381                  num_defects=num_defects,
 1382                  inference_time_ms=results.get('inference_time', 0),
 1383                  image_path=image_path,
 1384                  result_image_path=result_image_path,
 1385                  defects=results.get('detections', []),
 1386              )
 1387  
 1388          # SN码消费后清空
 1389          if self._current_sn:
 1390              self._current_sn = ""
 1391  
 1392          self.display_frame(vis_frame)
 1393          self.update_result_info(results)
 1394          self.update_statistics(results)
 1395  
 1396      def on_error(self, error_msg):
 1397          logger.error(error_msg)
 1398          self.log_text.append(f"[ERROR] {error_msg}")
 1399  
 1400      def on_camera_lost(self):
 1401          self.camera_status_label.setText("摄像头: 已断开 (尝试重连中...)")
 1402          self.camera_status_label.setStyleSheet("font-size: 12px; padding: 3px; color: #EF4444; background-color: #FEF2F2;")
 1403          self.log_text.append("[WARNING] 摄像头连接已断开，正在尝试重连...")
 1404  
 1405      def on_camera_restored(self):
 1406          self.camera_status_label.setText("摄像头: 已连接")
 1407          self.camera_status_label.setStyleSheet("font-size: 12px; padding: 3px; color: #10B981; background-color: #F0FDF4;")
 1408          self.log_text.append("[INFO] 摄像头连接已恢复")
 1409  
 1410      def update_statistics(self, results):
 1411          self._total_detections += 1
 1412          num_defects = results.get('num_detections', 0)
 1413  
 1414          if num_defects == 0:
 1415              self._total_pass += 1
 1416          else:
 1417              self._total_fail += 1
 1418  
 1419          for det in results.get('detections', []):
 1420              class_name = det.get('class_name', 'unknown')
 1421              self._defect_counter[class_name] += 1
 1422  
 1423          inference_time = results.get('inference_time', 0)
 1424          if inference_time > 0:
 1425              self._inference_times.append(inference_time)
 1426  
 1427          total = self._total_pass + self._total_fail
 1428          pass_rate = (self._total_pass / total * 100) if total > 0 else 0
 1429          self.pass_rate_widget.set_rate(pass_rate)
 1430          self.pass_rate_detail_label.setText(
 1431              f"合格: {self._total_pass} | 缺陷: {self._total_fail} | 总计: {total}"
 1432          )
 1433  
 1434          self.defect_bar_widget.set_data(dict(self._defect_counter))
 1435  
 1436          if self._inference_times:
 1437              avg_time = sum(self._inference_times) / len(self._inference_times)
 1438              max_time = max(self._inference_times)
 1439              min_time = min(self._inference_times)
 1440              self.avg_inference_label.setText(f"平均推理耗时: {avg_time:.1f} ms")
 1441              self.max_inference_label.setText(f"最大推理耗时: {max_time:.1f} ms")
 1442              self.min_inference_label.setText(f"最小推理耗时: {min_time:.1f} ms")
 1443  
 1444          self.total_detected_label.setText(f"累计检测帧数: {self._total_detections}")
 1445  
 1446      def export_csv(self):
 1447          """导出CSV报表"""
 1448          file_path, _ = QFileDialog.getSaveFileName(
 1449              self, "导出CSV报表", f"pcb_report_{datetime.now().strftime('%Y%m%d_%H%M%S')}.csv",
 1450              "CSV文件 (*.csv)"
 1451          )
 1452  
 1453          if not file_path:
 1454              return
 1455  
 1456          try:
 1457              with open(file_path, 'w', newline='', encoding='utf-8-sig') as f:
 1458                  writer = csv.writer(f)
 1459  
 1460                  writer.writerow(["PCB缺陷检测系统 - 检测报表"])
 1461                  writer.writerow(["导出时间", datetime.now().strftime("%Y-%m-%d %H:%M:%S")])
 1462                  writer.writerow(["产线", self.production_manager.current_profile_name])
 1463                  writer.writerow([])
 1464  
 1465                  writer.writerow(["=== 统计概要 ==="])
 1466                  total = self._total_pass + self._total_fail
 1467                  pass_rate = (self._total_pass / total * 100) if total > 0 else 0
 1468                  writer.writerow(["总检测帧数", self._total_detections])
 1469                  writer.writerow(["合格帧数", self._total_pass])
 1470                  writer.writerow(["缺陷帧数", self._total_fail])
 1471                  writer.writerow(["合格率", f"{pass_rate:.1f}%"])
 1472  
 1473                  if self._inference_times:
 1474                      avg_time = sum(self._inference_times) / len(self._inference_times)
 1475                      writer.writerow(["平均推理耗时(ms)", f"{avg_time:.1f}"])
 1476                      writer.writerow(["最大推理耗时(ms)", f"{max(self._inference_times):.1f}"])
 1477                      writer.writerow(["最小推理耗时(ms)", f"{min(self._inference_times):.1f}"])
 1478                  writer.writerow([])
 1479  
 1480                  writer.writerow(["=== 缺陷类别分布 ==="])
 1481                  writer.writerow(["类别", "数量", "占比"])
 1482                  total_defects = sum(self._defect_counter.values())
 1483                  for cat, count in self._defect_counter.most_common():
 1484                      ratio = f"{count / total_defects * 100:.1f}%" if total_defects > 0 else "0%"
 1485                      writer.writerow([cat, count, ratio])
 1486                  writer.writerow([])
 1487  
 1488                  if self.expert_scores:
 1489                      writer.writerow(["=== 专家评分记录 ==="])
 1490                      writer.writerow(["序号", "时间", "检测数量", "评分"])
 1491                      for record in self.expert_scores:
 1492                          writer.writerow([
 1493                              record['index'], record['time'],
 1494                              record['detections'], "正确" if record['correct'] else "错误"
 1495                          ])
 1496  
 1497              QMessageBox.information(self, "导出成功", f"报表已保存至:\n{file_path}")
 1498              logger.info(f"CSV报表已导出: {file_path}")
 1499  
 1500          except Exception as e:
 1501              QMessageBox.critical(self, "导出失败", f"导出CSV失败:\n{str(e)}")
 1502  
 1503      def display_frame(self, frame):
 1504          if frame is None:
 1505              return
 1506  
 1507          rgb_frame = cv2.cvtColor(frame, cv2.COLOR_BGR2RGB)
 1508          h, w, ch = rgb_frame.shape
 1509          bytes_per_line = ch * w
 1510  
 1511          qt_image = QImage(rgb_frame.data, w, h, bytes_per_line, QImage.Format_RGB888)
 1512          pixmap = QPixmap.fromImage(qt_image)
 1513  
 1514          scaled_pixmap = pixmap.scaled(
 1515              self.video_label.size(),
 1516              Qt.KeepAspectRatio,
 1517              Qt.SmoothTransformation
 1518          )
 1519  
 1520          self.video_label.setPixmap(scaled_pixmap)
 1521  
 1522      def update_result_info(self, results):
 1523          self.fps_label.setText(f"{results.get('fps', 0):.1f}")
 1524          self.time_label.setText(f"{results.get('inference_time', 0):.1f} ms")
 1525          self.count_label.setText(f"{results.get('num_detections', 0)}")
 1526  
 1527          detections = results.get('detections', [])
 1528          if detections:
 1529              avg_conf = sum(d['confidence'] for d in detections) / len(detections)
 1530              self.conf_label.setText(f"{avg_conf:.3f}")
 1531          else:
 1532              self.conf_label.setText("--")
 1533  
 1534          self.result_table.setRowCount(len(detections))
 1535          for i, det in enumerate(detections):
 1536              self.result_table.setItem(i, 0, QTableWidgetItem(det.get('class_name', '')))
 1537              self.result_table.setItem(i, 1, QTableWidgetItem(f"{det.get('confidence', 0):.3f}"))
 1538              bbox = det.get('bbox', [0, 0, 0, 0])
 1539              bbox_str = f"({bbox[0]:.0f}, {bbox[1]:.0f}, {bbox[2]:.0f}, {bbox[3]:.0f})"
 1540              self.result_table.setItem(i, 2, QTableWidgetItem(bbox_str))
 1541  
 1542              # 添加"屏蔽此区域"按钮
 1543              suppress_btn = QPushButton("屏蔽")
 1544              suppress_btn.clicked.connect(lambda checked, b=bbox: self._quick_suppress(b))
 1545              self.result_table.setCellWidget(i, 3, suppress_btn)
 1546  
 1547      def _quick_suppress(self, bbox):
 1548          """快速屏蔽某个检测框区域"""
 1549          x1, y1, x2, y2 = map(int, bbox)
 1550          margin = 20  # 扩展一点区域
 1551          name = f"auto_{datetime.now().strftime('%H%M%S')}"
 1552          self.suppressor.add_rect_rule(
 1553              name=name,
 1554              x1=max(0, x1 - margin), y1=max(0, y1 - margin),
 1555              x2=x2 + margin, y2=y2 + margin,
 1556              note=f"自动屏蔽于{datetime.now().strftime('%H:%M:%S')}"
 1557          )
 1558          self.log_text.append(f"[INFO] 已添加屏蔽区域: {name}")
 1559  
 1560      def update_display(self):
 1561          pass
 1562  
 1563      def update_status(self):
 1564          if self.is_emergency_stopped:
 1565              status = "⛔ 急停"
 1566          elif self.is_running:
 1567              status = "运行中"
 1568          else:
 1569              status = "就绪"
 1570  
 1571          user_info = ""
 1572          if self.auth_manager.current_user:
 1573              user_info = f" | {self.auth_manager.current_user.display_name}"
 1574  
 1575          self.status_label.setText(
 1576              f"状态: {status} | 历史记录: {len(self.history)}{user_info}"
 1577          )
 1578  
 1579      def capture_image(self):
 1580          if self.current_frame is None:
 1581              return
 1582  
 1583          save_dir = Path(self.config.system.save_dir)
 1584          save_dir.mkdir(parents=True, exist_ok=True)
 1585  
 1586          timestamp = datetime.now().strftime("%Y%m%d_%H%M%S")
 1587          save_path = save_dir / f"capture_{timestamp}.jpg"
 1588  
 1589          cv2.imwrite(str(save_path), self.current_frame)
 1590          logger.info(f"截图已保存: {save_path}")
 1591          self.log_text.append(f"[INFO] 截图已保存: {save_path}")
 1592  
 1593      def load_image(self):
 1594          file_path, _ = QFileDialog.getOpenFileName(
 1595              self, "选择图片", "",
 1596              "Images (*.jpg *.jpeg *.png *.bmp)"
 1597          )
 1598  
 1599          if file_path:
 1600              if self.detector:
 1601                  results, vis_image = self.detector.detect_image(file_path, show=False)
 1602                  if vis_image is not None:
 1603                      self.display_frame(vis_image)
 1604                      self.update_result_info(results)
 1605                      self.update_statistics(results)
 1606                      self.current_frame = vis_image
 1607                      logger.info(f"已加载图片: {file_path}")
 1608  
 1609      def score_detection(self, is_correct):
 1610          if self.detection_results is None:
 1611              return
 1612  
 1613          self.total_images += 1
 1614          if is_correct:
 1615              self.correct_images += 1
 1616  
 1617          accuracy = self.correct_images / max(self.total_images, 1) * 100
 1618          self.score_label.setText(f"评分统计: {self.correct_images}/{self.total_images} ({accuracy:.1f}%)")
 1619  
 1620          score_record = {
 1621              'index': self.total_images,
 1622              'time': datetime.now().strftime("%H:%M:%S"),
 1623              'detections': self.detection_results.get('num_detections', 0),
 1624              'correct': is_correct
 1625          }
 1626          self.expert_scores.append(score_record)
 1627  
 1628          row = self.score_table.rowCount()
 1629          self.score_table.insertRow(row)
 1630          self.score_table.setItem(row, 0, QTableWidgetItem(str(score_record['index'])))
 1631          self.score_table.setItem(row, 1, QTableWidgetItem(score_record['time']))
 1632          self.score_table.setItem(row, 2, QTableWidgetItem(str(score_record['detections'])))
 1633          self.score_table.setItem(row, 3, QTableWidgetItem("✓" if is_correct else "✗"))
 1634  
 1635      def closeEvent(self, event):
 1636          self.stop_detection()
 1637          self.trace_db.close()
 1638          event.accept()
 1639  
 1640  
 1641  def main():
 1642      """主函数"""
 1643      import argparse as _argparse
 1644  
 1645      _parser = _argparse.ArgumentParser(description="PCB缺陷检测系统GUI")
 1646      _parser.add_argument("--mock", action="store_true", help="使用模拟检测器（不需要真实模型）")
 1647      _args = _parser.parse_args()
 1648  
 1649      app = QApplication(sys.argv)
 1650      app.setStyle('Fusion')
 1651  
 1652      config = get_config()
 1653  
 1654      # 权限管理 - 登录
 1655      auth_manager = AuthManager(
 1656          db_path=getattr(config, 'auth', None) and config.auth.db_path or None
 1657      )
 1658  
 1659      if getattr(config, 'auth', None) and config.auth.enabled:
 1660          login_dialog = LoginDialog(auth_manager)
 1661          if login_dialog.exec_() != QDialog.Accepted:
 1662              sys.exit(0)
 1663  
 1664      # 创建检测器
 1665      detector = None
 1666      try:
 1667          if _args.mock:
 1668              logger.info("使用模拟检测器模式")
 1669              from pathlib import Path
 1670              _sys_path = sys.path.copy()
 1671              sys.path.insert(0, str(Path(__file__).parent.parent))
 1672              from demo_simulate import MockDetector
 1673              detector = MockDetector(config)
 1674              sys.path = _sys_path  # 恢复原路径
 1675          else:
 1676              from inference import PCBDefectDetector
 1677              detector = PCBDefectDetector(config)
 1678      except Exception as e:
 1679          logger.warning(f"检测器初始化失败: {e}")
 1680  
 1681      # 创建控制器
 1682      from utils.serial_control import create_controller
 1683      controller = create_controller(config, mock=True)
 1684      controller.connect()
 1685  
 1686      # 创建主窗口
 1687      window = MainWindow(config, detector, controller)
 1688      window.auth_manager = auth_manager
 1689      window.show()
 1690  
 1691      sys.exit(app.exec_())
 1692  
 1693  
 1694  if __name__ == "__main__":
 1695      main()
 1696  
 1697  # ============================================================
 1698  # 文件: demo_simulate.py
 1699  # 说明: 模拟检测器 — 无模型时的模拟推理
 1700  # 行数: 629
 1701  # ============================================================
 1702  # -*- coding: utf-8 -*-
 1703  """
 1704  PCB缺陷检测系统 — 模拟演示脚本
 1705  无需真实硬件、摄像头或训练好的模型即可运行
 1706  
 1707  功能：
 1708  1. 生成合成PCB图像（含模拟缺陷）
 1709  2. 使用yolo11n预训练权重进行推理（或模拟推理）
 1710  3. 模拟完整检测流程：采集→推理→决策→分拣
 1711  4. 展示所有6类缺陷的检测结果
 1712  5. 生成演示报告
 1713  """
 1714  
 1715  import os
 1716  import sys
 1717  import time
 1718  import json
 1719  import random
 1720  from pathlib import Path
 1721  from datetime import datetime
 1722  from collections import Counter
 1723  
 1724  import cv2
 1725  import numpy as np
 1726  
 1727  sys.path.append(os.path.dirname(os.path.abspath(__file__)))
 1728  from utils.config_loader import get_config
 1729  from utils.logger import setup_logger
 1730  from utils.serial_control import MockArduinoController
 1731  
 1732  logger = setup_logger("demo")
 1733  
 1734  
 1735  # ==================== 合成PCB图像生成 ====================
 1736  
 1737  CLASS_NAMES = ['missing_hole', 'mouse_bite', 'open_circuit', 'short_circuit', 'spur', 'spurious_copper']
 1738  CLASS_NAMES_CN = ['漏孔', '鼠咬', '开路', '短路', '毛刺', '多余铜']
 1739  CLASS_COLORS = [
 1740      (0, 0, 255),    # 红
 1741      (0, 165, 255),  # 橙
 1742      (0, 255, 255),  # 黄
 1743      (0, 255, 0),    # 绿
 1744      (255, 0, 255),  # 紫
 1745      (255, 255, 0),  # 青
 1746  ]
 1747  
 1748  
 1749  def generate_pcb_base(width=640, height=640):
 1750      """生成PCB底板图像（绿色基板+铜走线+焊盘+过孔）"""
 1751      # 绿色PCB基板
 1752      img = np.ones((height, width, 3), dtype=np.uint8) * 30
 1753      img[:, :, 1] = 120  # 绿色
 1754      img[:, :, 0] = 40
 1755      img[:, :, 2] = 30
 1756  
 1757      # 添加纹理噪点
 1758      noise = np.random.randint(-15, 15, (height, width, 3), dtype=np.int16)
 1759      img = np.clip(img.astype(np.int16) + noise, 0, 255).astype(np.uint8)
 1760  
 1761      # 绘制铜走线
 1762      for _ in range(random.randint(8, 15)):
 1763          x1 = random.randint(0, width)
 1764          y1 = random.randint(0, height)
 1765          angle = random.choice([0, 90, 45, 135])
 1766          length = random.randint(100, 400)
 1767          x2 = int(x1 + length * np.cos(np.radians(angle)))
 1768          y2 = int(y1 + length * np.sin(np.radians(angle)))
 1769          thickness = random.randint(2, 4)
 1770          cv2.line(img, (x1, y1), (x2, y2), (180, 160, 60), thickness)
 1771  
 1772      # 绘制焊盘
 1773      for _ in range(random.randint(10, 25)):
 1774          cx = random.randint(30, width - 30)
 1775          cy = random.randint(30, height - 30)
 1776          r = random.randint(6, 12)
 1777          cv2.circle(img, (cx, cy), r, (180, 160, 60), -1)
 1778          cv2.circle(img, (cx, cy), r // 2, (40, 120, 30), -1)
 1779  
 1780      # 绘制过孔
 1781      for _ in range(random.randint(5, 15)):
 1782          cx = random.randint(30, width - 30)
 1783          cy = random.randint(30, height - 30)
 1784          cv2.circle(img, (cx, cy), 4, (180, 160, 60), -1)
 1785          cv2.circle(img, (cx, cy), 2, (30, 30, 30), -1)
 1786  
 1787      # IC芯片
 1788      for _ in range(random.randint(1, 3)):
 1789          x = random.randint(50, width - 150)
 1790          y = random.randint(50, height - 100)
 1791          w = random.randint(60, 120)
 1792          h = random.randint(40, 80)
 1793          cv2.rectangle(img, (x, y), (x + w, y + h), (30, 30, 30), -1)
 1794          cv2.rectangle(img, (x, y), (x + w, y + h), (180, 160, 60), 1)
 1795          # 引脚
 1796          for i in range(0, w, 8):
 1797              cv2.line(img, (x + i, y), (x + i, y - 6), (180, 160, 60), 1)
 1798              cv2.line(img, (x + i, y + h), (x + i, y + h + 6), (180, 160, 60), 1)
 1799  
 1800      return img
 1801  
 1802  
 1803  def add_defect(img, defect_type, bbox_list):
 1804      """在PCB图像上添加指定类型的缺陷"""
 1805      h, w = img.shape[:2]
 1806      margin = 40
 1807  
 1808      if defect_type == 0:  # missing_hole (漏孔)
 1809          cx = random.randint(margin, w - margin)
 1810          cy = random.randint(margin, h - margin)
 1811          r = random.randint(8, 15)
 1812          # 画焊盘但没有孔
 1813          cv2.circle(img, (cx, cy), r, (180, 160, 60), -1)
 1814          # 漏孔：没有中心孔，添加一个标记点
 1815          cv2.circle(img, (cx, cy), 2, (200, 200, 200), -1)
 1816          bbox = [cx - r, cy - r, cx + r, cy + r]
 1817  
 1818      elif defect_type == 1:  # mouse_bite (鼠咬)
 1819          x = random.randint(margin, w - margin - 60)
 1820          y = random.randint(margin, h - margin)
 1821          # 画一条走线，然后咬掉一块
 1822          cv2.line(img, (x, y), (x + 80, y), (180, 160, 60), 3)
 1823          bite_w = random.randint(5, 12)
 1824          bite_h = random.randint(3, 8)
 1825          bx = x + random.randint(20, 50)
 1826          cv2.rectangle(img, (bx, y - bite_h), (bx + bite_w, y + 1), (40, 120, 30), -1)
 1827          bbox = [bx - 2, y - bite_h - 2, bx + bite_w + 2, y + 3]
 1828  
 1829      elif defect_type == 2:  # open_circuit (开路)
 1830          x = random.randint(margin, w - margin - 100)
 1831          y = random.randint(margin, h - margin)
 1832          cv2.line(img, (x, y), (x + 40, y), (180, 160, 60), 3)
 1833          gap = random.randint(8, 15)
 1834          cv2.line(img, (x + 40 + gap, y), (x + 100, y), (180, 160, 60), 3)
 1835          # 间隙处填充背景色
 1836          cv2.rectangle(img, (x + 40, y - 4), (x + 40 + gap, y + 4), (40, 120, 30), -1)
 1837          bbox = [x + 38, y - 6, x + 42 + gap, y + 6]
 1838  
 1839      elif defect_type == 3:  # short_circuit (短路)
 1840          x = random.randint(margin, w - margin - 80)
 1841          y1 = random.randint(margin + 10, h - margin - 20)
 1842          y2 = y1 + random.randint(15, 25)
 1843          cv2.line(img, (x, y1), (x + 80, y1), (180, 160, 60), 3)
 1844          cv2.line(img, (x, y2), (x + 80, y2), (180, 160, 60), 3)
 1845          # 短路桥
 1846          bridge_x = x + random.randint(20, 50)
 1847          cv2.line(img, (bridge_x, y1), (bridge_x, y2), (180, 160, 60), 2)
 1848          bbox = [bridge_x - 4, y1 - 4, bridge_x + 4, y2 + 4]
 1849  
 1850      elif defect_type == 4:  # spur (毛刺)
 1851          x = random.randint(margin, w - margin)
 1852          y = random.randint(margin, h - margin)
 1853          cv2.line(img, (x - 30, y), (x + 30, y), (180, 160, 60), 3)
 1854          # 毛刺
 1855          spur_len = random.randint(8, 18)
 1856          spur_angle = random.uniform(-60, 60)
 1857          sx = int(x + spur_len * np.cos(np.radians(spur_angle)))
 1858          sy = int(y + spur_len * np.sin(np.radians(spur_angle)))
 1859          cv2.line(img, (x, y), (sx, sy), (180, 160, 60), 2)
 1860          bbox = [min(x, sx) - 3, min(y, sy) - 3, max(x, sx) + 3, max(y, sy) + 3]
 1861  
 1862      elif defect_type == 5:  # spurious_copper (多余铜)
 1863          cx = random.randint(margin + 20, w - margin - 20)
 1864          cy = random.randint(margin + 20, h - margin - 20)
 1865          size = random.randint(8, 18)
 1866          # 多余铜块
 1867          pts = np.array([
 1868              [cx, cy - size],
 1869              [cx + size, cy],
 1870              [cx + size // 2, cy + size],
 1871              [cx - size // 2, cy + size // 2],
 1872          ], np.int32)
 1873          cv2.fillPoly(img, [pts], (180, 160, 60))
 1874          bbox = [cx - size - 2, cy - size - 2, cx + size + 2, cy + size + 2]
 1875  
 1876      else:
 1877          bbox = [0, 0, 10, 10]
 1878  
 1879      bbox_list.append({
 1880          'bbox': bbox,
 1881          'class_id': defect_type,
 1882          'class_name': CLASS_NAMES[defect_type],
 1883          'class_name_cn': CLASS_NAMES_CN[defect_type],
 1884      })
 1885      return img
 1886  
 1887  
 1888  def generate_sample_image(defect_types=None, width=640, height=640):
 1889      """生成一张含指定缺陷的PCB图像"""
 1890      img = generate_pcb_base(width, height)
 1891      bboxes = []
 1892  
 1893      if defect_types is None:
 1894          defect_types = [random.randint(0, 5) for _ in range(random.randint(1, 3))]
 1895  
 1896      for dt in defect_types:
 1897          img = add_defect(img, dt, bboxes)
 1898  
 1899      return img, bboxes
 1900  
 1901  
 1902  # ==================== 模拟检测器 ====================
 1903  
 1904  class MockDetector:
 1905      """模拟检测器（不需要真实模型）"""
 1906  
 1907      def __init__(self, config=None):
 1908          self.config = config or get_config()
 1909          self.class_names = CLASS_NAMES
 1910          self.class_colors = CLASS_COLORS
 1911          self.fps_history = []
 1912  
 1913      def detect(self, image, ground_truth=None):
 1914          """
 1915          模拟检测：如果有ground_truth则使用，否则随机生成检测结果
 1916          """
 1917          start_time = time.time()
 1918  
 1919          if ground_truth is not None:
 1920              # 有ground truth时：使用ground truth + 模拟置信度
 1921              # ground_truth为空列表时表示合格品，不生成检测
 1922              detections = []
 1923              for gt in ground_truth:
 1924                  conf = random.uniform(0.65, 0.98)
 1925                  # 模拟轻微的bbox偏移
 1926                  bbox = gt['bbox'].copy()
 1927                  offset = random.randint(-3, 3)
 1928                  bbox = [b + offset for b in bbox]
 1929                  detections.append({
 1930                      'bbox': bbox,
 1931                      'confidence': conf,
 1932                      'class_id': gt['class_id'],
 1933                      'class_name': gt['class_name'],
 1934                  })
 1935          else:
 1936              # 随机生成1-3个检测
 1937              num_det = random.randint(0, 3)
 1938              detections = []
 1939              for _ in range(num_det):
 1940                  cls_id = random.randint(0, 5)
 1941                  x1 = random.randint(50, 500)
 1942                  y1 = random.randint(50, 500)
 1943                  x2 = x1 + random.randint(15, 40)
 1944                  y2 = y1 + random.randint(15, 40)
 1945                  detections.append({
 1946                      'bbox': [x1, y1, x2, y2],
 1947                      'confidence': random.uniform(0.55, 0.95),
 1948                      'class_id': cls_id,
 1949                      'class_name': CLASS_NAMES[cls_id],
 1950                  })
 1951  
 1952          # 模拟推理耗时
 1953          inference_time = random.uniform(150, 400)
 1954          time.sleep(inference_time / 1000.0)  # 模拟真实耗时
 1955  
 1956          fps = 1000.0 / inference_time
 1957          self.fps_history.append(fps)
 1958          avg_fps = sum(self.fps_history[-30:]) / len(self.fps_history[-30:])
 1959  
 1960          return {
 1961              'detections': detections,
 1962              'num_detections': len(detections),
 1963              'inference_time': inference_time,
 1964              'fps': avg_fps,
 1965              'image_shape': image.shape[:2],
 1966              'backend': 'mock'
 1967          }
 1968  
 1969      def draw_results(self, image, results, show_cn=True):
 1970          """绘制检测结果（使用PIL支持中文）"""
 1971          from PIL import Image as PILImage, ImageDraw as PILDraw, ImageFont as PILFont
 1972  
 1973          img = image.copy()
 1974          pil_img = PILImage.fromarray(cv2.cvtColor(img, cv2.COLOR_BGR2RGB))
 1975          draw = PILDraw.Draw(pil_img)
 1976  
 1977          # 加载字体
 1978          font_paths = [
 1979              "C:/Windows/Fonts/msyh.ttc",
 1980              "C:/Windows/Fonts/simhei.ttf",
 1981              "C:/Windows/Fonts/simsun.ttc",
 1982          ]
 1983          font = None
 1984          font_small = None
 1985          for fp in font_paths:
 1986              if os.path.exists(fp):
 1987                  try:
 1988                      font = PILFont.truetype(fp, 18)
 1989                      font_small = PILFont.truetype(fp, 16)
 1990                      break
 1991                  except Exception:
 1992                      continue
 1993          if font is None:
 1994              font = PILFont.load_default()
 1995              font_small = font
 1996  
 1997          for det in results['detections']:
 1998              x1, y1, x2, y2 = map(int, det['bbox'])
 1999              cls_id = det['class_id']
 2000              conf = det['confidence']
 2001              label = det.get('class_name_cn', det['class_name']) if show_cn else det['class_name']
 2002              color = self.class_colors[cls_id]
 2003  
 2004              # 边界框
 2005              draw.rectangle([x1, y1, x2, y2], outline=color, width=2)
 2006  
 2007              # 标签背景
 2008              text = f"{label} {conf:.2f}"
 2009              bbox = draw.textbbox((x1, y1 - 22), text, font=font_small)
 2010              draw.rectangle(bbox, fill=color)
 2011              draw.text((x1, y1 - 22), text, fill=(255, 255, 255), font=font_small)
 2012  
 2013          # 信息栏
 2014          info_lines = [
 2015              f"FPS: {results['fps']:.1f}",
 2016              f"Time: {results['inference_time']:.1f}ms",
 2017              f"Defects: {results['num_detections']}",
 2018              f"Backend: {results['backend']}",
 2019          ]
 2020          for i, line in enumerate(info_lines):
 2021              draw.text((10, 10 + i * 24), line, fill=(0, 255, 0), font=font_small)
 2022  
 2023          return cv2.cvtColor(np.array(pil_img), cv2.COLOR_RGB2BGR)
 2024  
 2025  
 2026  # ==================== 演示流程 ====================
 2027  
 2028  def run_demo():
 2029      """运行模拟演示"""
 2030      logger.info("=" * 60)
 2031      logger.info("PCB缺陷检测与智能分拣系统 — 模拟演示")
 2032      logger.info("=" * 60)
 2033  
 2034      config = get_config()
 2035      detector = MockDetector(config)
 2036      controller = MockArduinoController()
 2037  
 2038      # 创建输出目录
 2039      save_dir = Path("results/demo")
 2040      save_dir.mkdir(parents=True, exist_ok=True)
 2041  
 2042      # 统计
 2043      total = 0
 2044      ok_count = 0
 2045      ng_count = 0
 2046      defect_counter = Counter()
 2047      all_results = []
 2048  
 2049      logger.info("\n阶段1: 生成合成PCB样本并检测")
 2050      logger.info("-" * 40)
 2051  
 2052      # 生成20张样本（模拟竞赛评估）
 2053      sample_configs = [
 2054          # 每张图指定缺陷类型
 2055          [0], [1], [2], [3], [4], [5],           # 每种缺陷各一张
 2056          [0, 1], [2, 3], [4, 5],                  # 组合缺陷
 2057          [0, 0], [1, 2], [3, 4],                  # 更多组合
 2058          [5, 0], [2, 4], [1, 3, 5],               # 三缺陷
 2059          [], [], [], [], [], [], []                # 合格品（7张）
 2060      ]
 2061  
 2062      for i, defect_types in enumerate(sample_configs):
 2063          logger.info(f"\n样本 {i + 1}/20")
 2064  
 2065          # 1. 生成PCB图像
 2066          img, ground_truth = generate_sample_image(defect_types)
 2067  
 2068          # 2. 模拟采集（保存原始图）
 2069          raw_path = save_dir / f"raw_{i + 1:02d}.jpg"
 2070          cv2.imwrite(str(raw_path), img)
 2071  
 2072          # 3. 模型推理
 2073          results = detector.detect(img, ground_truth=ground_truth)
 2074  
 2075          # 4. 判定结果
 2076          is_defective = results['num_detections'] > 0
 2077          total += 1
 2078  
 2079          if is_defective:
 2080              ng_count += 1
 2081              for det in results['detections']:
 2082                  defect_counter[det['class_name']] += 1
 2083              logger.info(f"  判定: 不合格 (检测到 {results['num_detections']} 个缺陷)")
 2084              controller.sort_pcb(is_ok=False)
 2085          else:
 2086              ok_count += 1
 2087              logger.info(f"  判定: 合格")
 2088              controller.sort_pcb(is_ok=True)
 2089  
 2090          # 5. 绘制结果
 2091          vis_img = detector.draw_results(img, results)
 2092          result_path = save_dir / f"result_{i + 1:02d}.jpg"
 2093          cv2.imwrite(str(result_path), vis_img)
 2094  
 2095          all_results.append({
 2096              'index': i + 1,
 2097              'defects': results['num_detections'],
 2098              'inference_time': results['inference_time'],
 2099              'is_defective': is_defective,
 2100              'detection_details': [{
 2101                  'class_name': d['class_name'],
 2102                  'class_name_cn': CLASS_NAMES_CN[d['class_id']],
 2103                  'confidence': d['confidence'],
 2104              } for d in results['detections']],
 2105          })
 2106  
 2107      # 生成评估报告
 2108      logger.info("\n" + "=" * 60)
 2109      logger.info("演示评估报告")
 2110      logger.info("=" * 60)
 2111  
 2112      pass_rate = ok_count / total * 100 if total > 0 else 0
 2113      avg_time = sum(r['inference_time'] for r in all_results) / len(all_results)
 2114  
 2115      logger.info(f"总样本数: {total}")
 2116      logger.info(f"合格品: {ok_count}")
 2117      logger.info(f"不合格品: {ng_count}")
 2118      logger.info(f"合格率: {pass_rate:.1f}%")
 2119      logger.info(f"平均推理耗时: {avg_time:.1f}ms")
 2120      logger.info(f"平均FPS: {1000 / avg_time:.1f}")
 2121  
 2122      logger.info("\n缺陷类别分布:")
 2123      for name, count in defect_counter.most_common():
 2124          cn_name = CLASS_NAMES_CN[CLASS_NAMES.index(name)]
 2125          logger.info(f"  {name} ({cn_name}): {count}")
 2126  
 2127      # 保存JSON报告
 2128      report = {
 2129          'timestamp': datetime.now().isoformat(),
 2130          'total_samples': total,
 2131          'pass_count': ok_count,
 2132          'fail_count': ng_count,
 2133          'pass_rate': pass_rate,
 2134          'avg_inference_time_ms': avg_time,
 2135          'defect_distribution': dict(defect_counter),
 2136          'results': all_results,
 2137      }
 2138  
 2139      report_path = save_dir / 'demo_report.json'
 2140      with open(report_path, 'w', encoding='utf-8') as f:
 2141          json.dump(report, f, ensure_ascii=False, indent=2)
 2142  
 2143      logger.info(f"\n报告已保存: {report_path}")
 2144      logger.info(f"检测图片已保存: {save_dir}/")
 2145  
 2146      # 生成汇总图
 2147      generate_summary_image(save_dir, all_results, ok_count, ng_count, defect_counter, avg_time)
 2148  
 2149      logger.info("\n模拟演示完成！")
 2150      return report
 2151  
 2152  
 2153  def generate_summary_image(save_dir, results, ok_count, ng_count, defect_counter, avg_time):
 2154      """生成汇总展示图（使用PIL支持中文）"""
 2155      from PIL import Image, ImageDraw, ImageFont
 2156  
 2157      w, h = 1200, 800
 2158      img = Image.new('RGB', (w, h), (245, 245, 245))
 2159      draw = ImageDraw.Draw(img)
 2160  
 2161      # 尝试加载中文字体
 2162      font_paths = [
 2163          "C:/Windows/Fonts/msyh.ttc",    # 微软雅黑
 2164          "C:/Windows/Fonts/simhei.ttf",   # 黑体
 2165          "C:/Windows/Fonts/simsun.ttc",   # 宋体
 2166      ]
 2167      font_large = None
 2168      font_medium = None
 2169      font_small = None
 2170      for fp in font_paths:
 2171          if os.path.exists(fp):
 2172              try:
 2173                  font_large = ImageFont.truetype(fp, 28)
 2174                  font_medium = ImageFont.truetype(fp, 20)
 2175                  font_small = ImageFont.truetype(fp, 16)
 2176                  break
 2177              except Exception:
 2178                  continue
 2179      if font_large is None:
 2180          font_large = ImageFont.load_default()
 2181          font_medium = font_large
 2182          font_small = font_large
 2183  
 2184      # 标题
 2185      draw.text((30, 20), "PCB缺陷检测与智能分拣系统 — 演示报告", fill=(30, 30, 30), font=font_large)
 2186  
 2187      # 统计信息
 2188      y = 80
 2189      total = ok_count + ng_count
 2190      pass_rate = ok_count / total * 100 if total > 0 else 0
 2191      stats = [
 2192          f"总样本数: {total}",
 2193          f"合格品: {ok_count}  |  不合格品: {ng_count}",
 2194          f"合格率: {pass_rate:.1f}%",
 2195          f"平均推理耗时: {avg_time:.1f}ms  |  平均FPS: {1000 / avg_time:.1f}",
 2196      ]
 2197      for s in stats:
 2198          draw.text((30, y), s, fill=(50, 50, 50), font=font_medium)
 2199          y += 32
 2200  
 2201      # 缺陷分布柱状图
 2202      y = 240
 2203      draw.text((30, y), "缺陷类别分布:", fill=(30, 30, 30), font=font_medium)
 2204      y += 38
 2205  
 2206      max_count = max(defect_counter.values()) if defect_counter else 1
 2207      bar_max_w = 300
 2208      for i, name in enumerate(CLASS_NAMES):
 2209          cn_name = CLASS_NAMES_CN[i]
 2210          count = defect_counter.get(name, 0)
 2211          bar_w = int(count / max_count * bar_max_w) if max_count > 0 else 0
 2212  
 2213          # 标签
 2214          draw.text((30, y + 2), f"{cn_name}({name})", fill=(50, 50, 50), font=font_small)
 2215  
 2216          # 柱子
 2217          color = CLASS_COLORS[i]
 2218          draw.rectangle([220, y, 220 + bar_w, y + 22], fill=color)
 2219  
 2220          # 数值
 2221          draw.text((225 + bar_w, y + 2), str(count), fill=(50, 50, 50), font=font_small)
 2222          y += 32
 2223  
 2224      # 展示4张检测结果缩略图
 2225      thumb_size = 180
 2226      thumb_y = 80
 2227      result_files = sorted(save_dir.glob("result_*.jpg"))
 2228  
 2229      selected = []
 2230      for rf in result_files:
 2231          if len(selected) >= 4:
 2232              break
 2233          idx = int(rf.stem.split('_')[1]) - 1
 2234          if idx < len(results) and results[idx]['defects'] > 0:
 2235              selected.append(rf)
 2236  
 2237      x_offset = 550
 2238      for i, rf in enumerate(selected[:4]):
 2239          thumb = Image.open(str(rf)).resize((thumb_size, thumb_size))
 2240          row = i // 2
 2241          col = i % 2
 2242          tx = x_offset + col * (thumb_size + 20)
 2243          ty = thumb_y + row * (thumb_size + 20)
 2244          img.paste(thumb, (tx, ty))
 2245  
 2246      # 保存
 2247      summary_path = save_dir / 'summary.jpg'
 2248      img.save(str(summary_path), quality=95)
 2249      logger.info(f"汇总图已保存: {summary_path}")
 2250  
 2251  
 2252  # ==================== 交互式演示 ====================
 2253  
 2254  def run_interactive_demo():
 2255      """交互式模拟演示（OpenCV窗口）"""
 2256      logger.info("=" * 60)
 2257      logger.info("PCB缺陷检测 — 交互式模拟演示")
 2258      logger.info("按 [空格] 检测新样本 | [1-6] 添加指定缺陷 | [0] 合格品 | [q] 退出")
 2259      logger.info("=" * 60)
 2260  
 2261      config = get_config()
 2262      detector = MockDetector(config)
 2263      controller = MockArduinoController()
 2264  
 2265      total = 0
 2266      ok_count = 0
 2267      ng_count = 0
 2268  
 2269      # 初始显示
 2270      img, gt = generate_sample_image([0])
 2271      results = detector.detect(img, ground_truth=gt)
 2272      vis = detector.draw_results(img, results)
 2273      cv2.imshow("PCB Defect Detection - Interactive Demo", vis)
 2274  
 2275      while True:
 2276          key = cv2.waitKey(0) & 0xFF
 2277  
 2278          if key == ord('q'):
 2279              break
 2280          elif key == ord(' '):
 2281              # 随机检测
 2282              defects = [random.randint(0, 5) for _ in range(random.randint(0, 3))]
 2283              img, gt = generate_sample_image(defects)
 2284          elif key == ord('0'):
 2285              img, gt = generate_sample_image([])
 2286          elif ord('1') <= key <= ord('6'):
 2287              defect_type = key - ord('1')
 2288              img, gt = generate_sample_image([defect_type])
 2289          else:
 2290              continue
 2291  
 2292          # 检测
 2293          results = detector.detect(img, ground_truth=gt)
 2294          total += 1
 2295  
 2296          if results['num_detections'] > 0:
 2297              ng_count += 1
 2298              controller.sort_pcb(is_ok=False)
 2299          else:
 2300              ok_count += 1
 2301              controller.sort_pcb(is_ok=True)
 2302  
 2303          # 绘制
 2304          vis = detector.draw_results(img, results)
 2305  
 2306          # 添加统计信息
 2307          pass_rate = ok_count / total * 100 if total > 0 else 0
 2308          cv2.putText(vis, f"Total: {total}  Pass: {ok_count}  Fail: {ng_count}  Rate: {pass_rate:.1f}%",
 2309                      (10, vis.shape[0] - 20), cv2.FONT_HERSHEY_SIMPLEX, 0.6, (255, 255, 255), 2)
 2310  
 2311          cv2.imshow("PCB Defect Detection - Interactive Demo", vis)
 2312  
 2313      cv2.destroyAllWindows()
 2314      logger.info(f"交互演示结束: 检测{total}张, 合格{ok_count}, 不合格{ng_count}")
 2315  
 2316  
 2317  if __name__ == "__main__":
 2318      import argparse
 2319  
 2320      parser = argparse.ArgumentParser(description='PCB缺陷检测模拟演示')
 2321      parser.add_argument('--mode', type=str, default='auto',
 2322                          choices=['auto', 'interactive'],
 2323                          help='auto=自动20张样本, interactive=交互式')
 2324  
 2325      args = parser.parse_args()
 2326  
 2327      if args.mode == 'auto':
 2328          run_demo()
 2329      else:
 2330          run_interactive_demo()
 2331  
 2332  # ============================================================
 2333  # 文件: download_model.py
 2334  # 说明: 模型自动下载
 2335  # 行数: 71
 2336  # ============================================================
 2337  
 2338  # -*- coding: utf-8 -*-
 2339  """
 2340  下载 YOLO11s 预训练模型
 2341  """
 2342  import ssl
 2343  import urllib.request
 2344  from pathlib import Path
 2345  
 2346  MODEL_URL = "https://github.com/ultralytics/assets/releases/download/v8.3.0/yolo11s.pt"
 2347  MODEL_DIR = Path("models")
 2348  MODEL_DIR.mkdir(exist_ok=True)
 2349  MODEL_PATH = MODEL_DIR / "yolo11s.pt"
 2350  
 2351  
 2352  def download():
 2353      print(f"正在下载 YOLO11s 预训练模型...")
 2354      print(f"目标地址: {MODEL_URL}")
 2355      print(f"保存路径: {MODEL_PATH.absolute()}")
 2356  
 2357      if MODEL_PATH.exists():
 2358          print(f"模型已存在: {MODEL_PATH}")
 2359          return True
 2360  
 2361      try:
 2362          ssl_context = ssl.create_default_context()
 2363          ssl_context.check_hostname = True
 2364          ssl_context.verify_mode = ssl.CERT_REQUIRED
 2365  
 2366          with urllib.request.urlopen(MODEL_URL, context=ssl_context, timeout=60) as resp:
 2367              total_size = int(resp.headers.get('Content-Length', 0))
 2368              downloaded = 0
 2369              block_size = 8192
 2370              last_print = 0
 2371  
 2372              with open(MODEL_PATH, 'wb') as f:
 2373                  while True:
 2374                      data = resp.read(block_size)
 2375                      if not data:
 2376                          break
 2377                      f.write(data)
 2378                      downloaded += len(data)
 2379  
 2380                      if total_size > 0:
 2381                          progress = downloaded / total_size * 100
 2382                          if progress - last_print >= 10:
 2383                              last_print = progress
 2384                              print(f"下载进度: {progress:.1f}% ({downloaded / 1024 / 1024:.1f} MB)")
 2385  
 2386          if MODEL_PATH.exists():
 2387              print(f"\n✅ 下载成功! 模型大小: {MODEL_PATH.stat().st_size / 1024 / 1024:.1f} MB")
 2388              return True
 2389          else:
 2390              print("❌ 下载失败: 模型文件未生成")
 2391              return False
 2392  
 2393      except Exception as e:
 2394          print(f"❌ 下载失败: {str(e)}")
 2395          import traceback
 2396          traceback.print_exc()
 2397          return False
 2398  
 2399  
 2400  if __name__ == "__main__":
 2401      success = download()
 2402      if success:
 2403          print("\n🎉 现在你可以运行正常模式的 GUI 了!")
 2404          print("   运行命令: python -c \"import sys; sys.path.insert(0, '.'); from ui.gui import main; sys.argv=['ui.gui']; main()\"")
 2405      else:
 2406          print("\n💡 可以继续使用 --mock 参数运行模拟模式")
 2407